# NASA Aircraft Operations Divison Safety Brief

In case of fire or medical emergency dial x33333 (JSC Emergency Ops Center) from a house phone. State your name, exact location, and brief nature of emergency.

Cell phone number 281-483-3333

Security Policies:

- Personal vehicles will be parked outside Gate 17/18 and NASA 990 Gate.

- Access to NASA South facilities will be through Gate 17/18. Access to NASA 990 area will be through the turnstile gate at the northwest corner of hangar 990.

- Speed limit is 10mph inside the gate.

- No driving or parking of personal vehicles around hangars, ramp area, or flight line.

Hangar Hazards: This is an active flight operations facility.

- Be aware of maintenance personnel and related activities going on around you.

- Use the yellow walkway inside the hangars when transiting through the hangar.

- Do not access the personnel doors if the hangar doors are being opened or closed.

- Watch out and give way to ramp equipment (tractors, fork lifts, etc.); high noise environment may inhibit audible warnings.

PLEASE REMAIN WITHIN THE DESIGNATED FACILITY AREAS ASSIGNED FOR YOUR OPERATIONS (Hangar 135 / Hangar 990 / Building E994 / Designated ramp areas.

Flight Line Hazards:

- Jet exhaust can be hazardous up to 200 feet behind aircraft.

- Jet intake can be hazardous within 25 feet.

- Secure all loose items, as even small articles can enter intakes and destroy jet engines.

- All tugs, forklifts, aircraft being towed or taxing have the right of way.

Other Common Hazards:

- Aircraft support equipment being towed into and out of hangars.

- Aircraft on jacks.

- Sharp surfaces on aircraft (flaps/trailing edges of wings/gear doors).

- Trips - air hoses, power cables, grounding wires, etc.

- Slip hazards - fuel, hydraulic fluid, water, grease, oil, etc.

Foreign Object Debris (FOD):

- You are responsible for every item you bring onto this facility (inside the gate).

- Keep up with your stuff – check pockets/loose gear/jewelry/security badges/etc.

Tool Control:

- Report all missing tools immediately.

- Operate from your own operations tool box/kit.

Lightning Hold Procedures (Automatic Detection System activates when lightning is within 6 miles of the airfield):

- Discontinue Operations when:

  - You hear seven 1 second horn blast

  - Amber beacon illuminates and rotates

- Continue Operations when:

  - You hear 1 continuous horn blast for 7 seconds.

  - Amber beacon secures

# DO’S

- When you open an airfield gate, make sure it’s closed before you leave the gate.

- Keep your badge with you at all times while at NASA.

- In case of a hangar or building fire, evacuate the area, stay together.

- Look before crossing the flight line road on the east side of hangar 990.

- Wear hearing protection while on the flight line.

- Be careful while walking on the hangar floor. It is painted and can be slippery when wet.

# DON’TS

- Go on a self-guided tour (explore the area) outside your work area without permission, and an NASA AOD escort.

- Climb on any of the AOD aircraft or support equipment.

- Touch any of the hangar controls (valves or switches).

- Try to fight any building or hangar fires.

- Wear open toed footwear (sandals or flip flops), or inappropriate for an industrial facility.

- Get close to any AOD aircraft being worked on by aircraft mechanics.

Phone Numbers:

- Safety Office – 49609/46533

- Operations Duty Office – 47256

- Aircraft Maintenance Control – 49612

## Emergency Number:   Ext.  3-3333

Or the RED hotline phone in the hangar.  Do not use 911 (except as last resort).

Summer Time Specific:

- Heat Stress:

  - Stay hydrated. Drink plenty of fluids (water). Avoid drinks with caffeine.

  - Take frequent breaks inside/under the shade to cool off

  - Wear light, loose, breathable clothes

THINK – Take an extra minute for safety – be aware of what is going on around you and make

the effort to keep yourself and co-workers safe.