PROFILE - A PROGRAM TO GENERATE PROFILES FROM

HARPO/HARPA ENVIRONMENTAL MODELS

J.A. Harlan

T.M. Georges

R. Michael Jones

NOAA TM ERL WPL-
198

NOTICE

Mention of a commercial company or product does

not constitute an endorsement by NOAA

Environmental Research Laboratories.  Use for

publicity or advertising purposes, of information

from this publication concerning proprietary

products or the test of such products, is not

authorized.  
                            CONTENTS

PAGE

1.  WHAT PROFILE DOES. . . . . . . . . . . . . . . . . . . . . .1

1.1  Introduction . . . . . . . . . . . . . . . . . . . . .1

1.2  Note for Non-PC Users and PC-Users

without MicroSoft Compiler/Linker. . . . . . . . . .2

2.   INSTALLATION. . . . . . . . . . . . . . . . . . . . . . . .2

2.1  Installing PROFILE via the

Distribution Disk. . . . . . . . . . . . . . . . . .2

2.2  Installing PROFILE via Internet. . . . . . . . . . . .3

3.  THE SAMPLE CASE. . . . . . . . . . . . . . . . . . . . . . .4

3.1  Description of the Input and Output . . . . . . . . . .4

3.2  How to Run the Sample Case. . . . . . . . . . . . . . 21

4.  HOW TO USE PROFILE . . . . . . . . . . . . . . . . . . . . 21

4.1    Create an Input Data File . . . . . . . . . . . . . 21

4.2    The Two Modes for Running PROFILE . . . . . . . . . 22

4.2.1  Using Filenames other than DINP

with RUNPROF.BAT. . . . . . . . . . . . . . . . . 23

4.3  Compiling . . . . . . . . . . . . . . . . . . . . . . 24

5.  HOW PROFILE WORKS. . . . . . . . . . . . . . . . . . . . . 24

5.1  Basic Overview. . . . . . . . . . . . . . . . . . . . 24

5.2  The DINP File and Linking the

PROFILE Program . . . . . . . . . . . . . . . . . . 25

5.3  How FINDMODL Works. . . . . . . . . . . . . . . . . . 26

6.  References . . . . . . . . . . . . . . . . . . . . . . . . 26

PAGE

Appendix A:  Instructions for Non-PC Users and

PC-Users without MicroSoft

Compiler/Linker Utilities . . . . . . . . . . . 28

Appendix B:  Description of the Distribution Disk Contents . . 32

Appendix C:  Blank Input Data Forms. . . . . . . . . . . . . . 34

Appendix C1. Input Data Form for Sound Speed/Current

Speed Profiles. . . . . . . . . . . . . . . . . 35

Appendix C2. Input Data Form for Temperature Profiles. . . . . 36

Appendix C3. Input Data Form for Topography Profiles . . . . . 37

Appendix C4. Input Data Format for Tabular Data Sets . . . . . 38

Appendix C5. Table of DINP Values to Specify Profile Type. . . 39

Appendix D:  Linker Response File for the Sample Case. . . . . 40

Appendix E:  Linker Instructions Generated for the

Sample Case.. . . . . . . . . . . . . . . . . . 41

Appendix F:  Historical Note on the Use of Double

Precision . . . . . . . . . . . . . . . . . . . 42

Appendix G:  PROFILE Plotting Commands . . . . . . . . . . . . 43

PROFILE - A PROGRAM TO GENERATE PROFILES FROM

HARPO/HARPA ENVIRONMENTAL MODELS

J.A. Harlan, T.M. Georges, and R.M. Jones

Abstract.  We describe a FORTRAN computer program,

PROFILE, that will generate plotting-command files for

profile plots of the environmental models (e.g. sound

speed, current or wind velocity, temperature) used with

the acoustic ray tracing programs HARPO (underwater

acoustics) and HARPA (atmospheric acoustics).

Profiles (plots of depth/height versus environmental

model parameter) can be made for any longitude-latitude

for models that depend on all three spatial dimensions.

A companion report describes an associated plotting

program that produces PostScript files as well as

screen graphics for PC-compatible computers.  An

additional companion report describes a computer

program that calculates contours of these environmental

model parameters.  Includes a floppy disk with computer

programs and a sample case.  Also included are

instructions for obtaining the programs (with updates)

via Internet.

1.  WHAT PROFILE DOES

1.1  Introduction

The ray tracing programs HARPA (Jones et al., 1986a) and

HARPO (Jones et al., 1986b; Georges et al., 1990) use a variety

of subroutines to specify sound speed, current/wind velocity,

temperature, etc., as functions of the three spatial coordinates

(longitude, latitude, and depth/height).  When adjusting the

parameters of such models to approximate measurements, it is

helpful to plot profiles that compare the models to be used for

ray tracing with measurements.  We describe here a program,

PROFILE, that produces such profile plots specifically for the

model subroutines used by the HARPA/HARPO ray tracing programs.

The user may specify the longitude and latitude of the

profile and also the desired height/depth interval.  In addition,

it is possible to specify one or more profiles (represented by a

set of points, possibly measurements) to superimpose on any of

the profile plots for comparison.  It is also possible to plot

multiple profiles on the same graph as long as the same set of

models is used (though parameters may differ).  It is not

possible to plot profiles from different models on the same

graph.

Profile creates a text output file TXTOUT that is read by

the program PSGRAPH (Harlan, et al., 1991a) that creates plots of

publication quality which can be used in journal papers,

presentations and reports to display the models used in ray

tracing.  Besides producing PostScript files PSGRAPH also

produces PC-compatible screen graphics.

An additional companion report (Harlan, et al., 1991b)

describes a computer program CONPLT, for calculating contours of

the environmental models.

The body of this report has been written for the user with

an IBM-PC-compatible computer.  Because the PROFILE program was

written in machine-independent FORTRAN, it can be used on any

machine, however, PC-specific peripheral programs and operating

system commands are necessary also (see Section 5).

1.2  Note for Non-PC Computer Users and PC-Users without

Microsoft Compiler/Linker

Be sure to read Appendix A, which explains the

system-dependent functions of the software provided on the

distribution disk.  This appendix is divided into sections for

each operating system for which the PROFILE program has been

successfully compiled.

2.  INSTALLATION

2.1 Installing PROFILE via the Distribution Disk

The distribution floppy disk (1.2 MB, 5-1/4" IBM-PC/AT

format) contains ASCII files, object files for the models, object

files for the PROFILE program, three executable files and a copy

of the report you are now reading.  See Appendix B for a complete

description of the files on the disk.

There are over 100 files provided on the diskette.  Most of

these files (about 80) are the environmental models source code

and object files.  Because the models are used by programs

besides PROFILE, e.g., CONPLT and PC-HARPO, the user may want to

locate the models in a directory which would be available to all

programs.  The INSTALL procedure described below allows the user

to put the models in either 1) a subdirectory (called "MODELS")

to the PROFILE working directory or 2) any existing directory

(including the PROFILE working directory).  If option 1 is

chosen, the MODELS subdirectory will be created by the INSTALL

program.

First, move to the directory in which you want to store and

run the PROFILE program (and its peripheral files), but not

necessarily the models files.  The main directory of the

distribution diskette will always be copied into the directory

from which INSTALL is invoked.

Second, invoke the INSTALL procedure.  Type:  A:INSTALL.

The user is prompted for answers and may restart the INSTALL

program if a change is desired.

The location of the models files is written to the

MODELDIR.DAT file.  This file is used by the FINDMODL program to

determine where the object files for the models are located when

linking.

If, at a later time, the user decides to put the models

files in a different directory, the MODELDIR.DAT file could be

edited to reflect the new location of the models object files.

This would avoid having to re-install the PROFILE diskette.

2.2 Installing PROFILE via Internet

The PROFILE program and its peripheral programs (i.e., the

contents of the distribution disk) can be obtained via Internet

and anonymous FTP.  The "anonymous" account contians a directory

"raytracing" which is divided into several subdirectories.  The

subdirectory "profile" contains the files which you will need.

To access these files via anonymous FTP through the Internet,

proceed as follows:

1.   Log on to a host at your site that has an Internet

connection and also supports the FTP command.

2.   Invoke FTP by entering the Internet address of the server at

our site:

ftp pooh.wpl.erl.gov

or

ftp 140.172.32.11

3.   Log in as "anonymous"

4.   For the password, enter your complete Internet address,

e.g., jah@node.her.thr.gov

5.   General instructions and information about the anonymous FTP

account at our site is available in a file named "README" in

the "pub" subdirectory.  To transfer this file to your local

host, first change to the "pub" directory:

cd pub

and then enter:

get README

Note that the name of the file is in uppercase.

6.   Change your current directory to the "raytracing/profile"

subdirectory by entering:

cd raytracing/profile

7.   To transfer information about the profile subdirectory to

your local host, enter:

get readme.prf

8.   To transfer those files in the profile subdirectory that you

are interested in, enter:

get filename

where "filename" is the name of a file that you want to transfer.

You may also retrieve multiple files using "mget".

9.   Change your current directory to the "raytracing/models" subdirectory

by entering:

cd ../models

10.  To transfer all of the files in the "raytracing/models" subdirectory,

enter:

mget *.*

9.   To return to your local host, enter:

quit

At this point, all of the files you will need to run PROFILE will be

on your local host.

3.  THE SAMPLE CASE

The sample case illustrates many of the features of PROFILE

and allows the user to verify that the program is working

correctly.

Some of the features illustrated by the sample case are

sound speed profiles, current/wind speed (magnitude and

component) profiles, a temperature profile, a bottom topography

profile, and comparison of measured profiles (points) with a

sound speed profile produced from a HARPO model.

3.1  Description of the Sample Case Input and Output

Figures 1 through 6 show completed input data forms

indicating the profiles desired and the models to be used.

(Appendices C1-C3 contain blank forms for general use.  Appendix

C4 describes the format for tabular input data).  There are two

examples of tabular data sets for sound speed profiles in the

sample case as well as one example for temperature profiles.

Because of the large number of models available, many

possible combinations of models can determine a profile.  Figure

7 shows the input data file corresponding to the completed input

data forms for the sample case (DINP.SAM).  Appendix G contains a

list of the plotting commands written to the output file, TXTOUT.

Figures 8 through 13 show the plots produced by the program,

PSGRAPH, for the sample case.

Figure 1.--Input Data Form for Sample Case, Plot 1, Sound speed

Profile

Figure 2.--Input Data Form for Sample Case, Plot 2, Current Speed

Profile

Figure 3.--Input Data Form for Sample Case, Plot 3, Southward

Current Component Profile

Figure 4.--Input Data Form for Sample Case, Plot 4, Eastward

Current Component Profile

Figure 5.--Input Data Form for Sample Case, Plot 5, Temperature

Profile

Figure 6.--Input Data Form for Sample Case, Plot 6, Topography

Profile

Figure 7.--Input Data File for the Sample Case

Figure 7.  (Cont.)

Figure 7.  (Cont.)

Figure 7.  (Cont.)

Figure 7.  (Cont.)

PROFILE - A PROGRAM TO GENERATE PROFILES FROM

HARPO/HARPA ENVIRONMENTAL MODELS

J.A. Harlan

T.M. Georges

R. Michael Jones

NOAA TM ERL WPL-
198

NOTICE

Mention of a commercial company or product does

not constitute an endorsement by NOAA

Environmental Research Laboratories.  Use for

publicity or advertising purposes, of information

from this publication concerning proprietary

products or the test of such products, is not

authorized.  
                            CONTENTS

PAGE

1.  WHAT PROFILE DOES. . . . . . . . . . . . . . . . . . . . . .1

1.1  Introduction . . . . . . . . . . . . . . . . . . . . .1

1.2  Note for Non-PC Users and PC-Users

without MicroSoft Compiler/Linker. . . . . . . . . .2

2.   INSTALLATION. . . . . . . . . . . . . . . . . . . . . . . .2

2.1  Installing PROFILE via the

Distribution Disk. . . . . . . . . . . . . . . . . .2

2.2  Installing PROFILE via Internet. . . . . . . . . . . .

3.  THE SAMPLE CASE. . . . . . . . . . . . . . . . . . . . . . .2

3.1  Description of the Input and Output . . . . . . . . . .3

3.2  How to Run the Sample Case. . . . . . . . . . . . . . .3

4.  HOW TO USE PROFILE . . . . . . . . . . . . . . . . . . . . .4

4.1    Create an Input Data File . . . . . . . . . . . . . .4

4.2    The Two Modes for Running PROFILE . . . . . . . . . .5

4.2.1  Using Filenames other than DINP

with RUNPROF.BAT. . . . . . . . . . . . . . . . . .6

4.3  Compiling . . . . . . . . . . . . . . . . . . . . . . .7

5.  HOW PROFILE WORKS. . . . . . . . . . . . . . . . . . . . . .7

5.1  Basic Overview. . . . . . . . . . . . . . . . . . . . .7

5.2  The DINP File and Linking the

PROFILE Program . . . . . . . . . . . . . . . . . . .8

5.3  How FINDMODL Works. . . . . . . . . . . . . . . . . . .9

6.  References . . . . . . . . . . . . . . . . . . . . . . . . .9

PAGE

Appendix A:  Instructions for Non-PC Users and

PC-Users without MicroSoft

Compiler/Linker Utilities . . . . . . . . . . . 11

Appendix B:  Description of the Distribution Disk Contents . . 15

Appendix C:  Blank Input Data Forms. . . . . . . . . . . . . . 17

Appendix C1. Input Data Form for Sound Speed/Current

Speed Profiles. . . . . . . . . . . . . . . . . 18

Appendix C2. Input Data Form for Temperature Profiles. . . . . 19

Appendix C3. Input Data Form for Topography Profiles . . . . . 20

Appendix C4. Input Data Format for Tabular Data Sets . . . . . 21

Appendix C5. Table of DINP Values to Specify Profile Type. . . 22

Appendix D:  Linker Response File for the Sample Case. . . . . 23

Appendix E:  Linker Instructions Generated for the

Sample Case.. . . . . . . . . . . . . . . . . . 24

Appendix F:  Historical Note on the Use of Double

Precision . . . . . . . . . . . . . . . . . . . 25

Appendix G:  PROFILE Plotting Commands . . . . . . . . . . . . 26

PROFILE - A PROGRAM TO GENERATE PROFILES FROM

HARPO/HARPA ENVIRONMENTAL MODELS

J.A. Harlan, T.M. Georges, and R.M. Jones

Abstract.  We describe a FORTRAN computer program,

PROFILE, that will generate plotting-command files for

profile plots of the environmental models (e.g. sound

speed, current or wind velocity, temperature) used with

the acoustic ray tracing programs HARPO (underwater

acoustics) and HARPA (atmospheric acoustics).

Profiles (plots of depth/height versus environmental

model parameter) can be made for any longitude-latitude

for models that depend on all three spatial dimensions.

A companion report describes an associated plotting

program that produces PostScript files as well as

screen graphics for PC-compatible computers.  An

additional companion report describes a computer

program that calculates contours of these environmental

model parameters.  Includes a floppy disk with computer

programs and a sample case.  Also included are

instructions for obtaining the programs (with updates)

via Internet.

1.  WHAT PROFILE DOES

1.1  Introduction

The ray tracing programs HARPA (Jones et al., 1986a) and

HARPO (Jones et al., 1986b; Georges et al., 1990) use a variety

of subroutines to specify sound speed, current/wind velocity,

temperature, etc., as functions of the three spatial coordinates

(longitude, latitude, and depth/height).  When adjusting the

parameters of such models to approximate measurements, it is

helpful to plot profiles that compare the models to be used for

ray tracing with measurements.  We describe here a program,

PROFILE, that produces such profile plots specifically for the

model subroutines used by the HARPA/HARPO ray tracing programs.

The user may specify the longitude and latitude of the

profile and also the desired height/depth interval.  In addition,

it is possible to specify one or more profiles (represented by a

set of points, possibly measurements) to superimpose on any of

the profile plots for comparison.  It is also possible to plot

multiple profiles on the same graph as long as the same set of

models is used (though parameters may differ).  It is not

possible to plot profiles from different models on the same

graph.

Profile creates a text output file TXTOUT that is read by

the program PSGRAPH (Harlan, et al., 1991a) that creates plots of

publication quality which can be used in journal papers,

presentations and reports to display the models used in ray

tracing.  Besides producing PostScript files PSGRAPH also

produces PC-compatible screen graphics.

An additional companion report (Harlan, et al., 1991b)

describes a computer program CONPLT, for calculating contours of

the environmental models.

The body of this report has been written for the user with

an IBM-PC-compatible computer.  Because the PROFILE program was

written in machine-independent FORTRAN, it can be used on any

machine, however, PC-specific peripheral programs and operating

system commands are necessary also (see Section 5).

1.2  Note for Non-PC Computer Users and PC-Users without

Microsoft Compiler/Linker

Be sure to read Appendix A, which explains the

system-dependent functions of the software provided on the

distribution disk.  This appendix is divided into sections for

each operating system for which the PROFILE program has been

successfully compiled.

2.  INSTALLATION

2.1 Installing PROFILE via the Distribution Disk

The distribution floppy disk (1.2 MB, 5-1/4" IBM-PC/AT

format) contains ASCII files, object files for the models, object

files for the PROFILE program, three executable files and a copy

of the report you are now reading.  See Appendix B for a complete

description of the files on the disk.

There are over 100 files provided on the diskette.  Most of

these files (about 80) are the environmental models source code

and object files.  Because the models are used by programs

besides PROFILE, e.g., CONPLT and PC-HARPO, the user may want to

locate the models in a directory which would be available to all

programs.  The INSTALL procedure described below allows the user

to put the models in either 1) a subdirectory (called "MODELS")

to the PROFILE working directory or 2) any existing directory

(including the PROFILE working directory).  If option 1 is

chosen, the MODELS subdirectory will be created by the INSTALL

program.

First, move to the directory in which you want to store and

run the PROFILE program (and its peripheral files), but not

necessarily the models files.  The main directory of the

distribution diskette will always be copied into the directory

from which INSTALL is invoked.

Second, invoke the INSTALL procedure.  Type:  A:INSTALL.

The user is prompted for answers and may restart the INSTALL

program if a change is desired.

The location of the models files is written to the

MODELDIR.DAT file.  This file is used by the FINDMODL program to

determine where the object files for the models are located when

linking.

If, at a later time, the user decides to put the models

files in a different directory, the MODELDIR.DAT file could be

edited to reflect the new location of the models object files.

This would avoid having to re-install the PROFILE diskette.

2.2 Installing PROFILE via Internet

The PROFILE program and its peripheral programs (i.e., the

contents of the distribution disk) can be obtained via Internet

and anonymous FTP.  The "anonymous" account contians a directory

"raytracing" which is divided into several subdirectories.  The

subdirectory "profile" contains the files which you will need.

To access these files via anonymous FTP through the Internet,

proceed as follows:

1.   Log on to a host at your site that has an Internet

connection and also supports the FTP command.

2.   Invoke FTP by entering the Internet address of the server at

our site:

ftp pooh.wpl.erl.gov

or

ftp 140.172.32.11

3.   Log in as "anonymous"

4.   For the password, enter your complete Internet address,

e.g., jah@node.her.thr.gov

5.   General instructions and information about the anonymous FTP

account at our site is available in a file named "readme."

To transfer this file to your local host, enter:

get readme

6.   Change your current directory to the "raytracing/profile"

subdirectory by entering:

cd raytracing/profile

7.   To transfer information about the profile subdirectory to

your local host, enter:

get readme.prf

8.   To transfer those files in the profile subdirectory that you

are interested in, enter:

get file name

9.   To return to your local host, enter:

quit

3.  THE SAMPLE CASE

The sample case illustrates many of the features of PROFILE

and allows the user to verify that the program is working

correctly.

Some of the features illustrated by the sample case are

sound speed profiles, current/wind speed (magnitude and

component) profiles, a temperature profile, a bottom topography

profile, and comparison of measured profiles (points) with a

sound speed profile produced from a HARPO model.

3.1  Description of the Sample Case Input and Output

Figures 1 through 6 show completed input data forms

indicating the profiles desired and the models to be used.

(Appendices C1-C3 contain blank forms for general use.  Appendix

C4 describes the format for tabular input data).  There are two

examples of tabular data sets for sound speed profiles in the

sample case as well as one example for temperature profiles.

Because of the large number of models available, many

possible combinations of models can determine a profile.  Figure

7 shows the input data file corresponding to the completed input

data forms for the sample case (DINP.SAM).  Appendix G contains a

list of the plotting commands written to the output file, TXTOUT.

Figures 8 through 13 show the plots produced by the program,

PSGRAPH, for the sample case.

3.2  How to Run the Sample Case

In this section, boldface upper case letters indicate user

input.

Type:  RUNPROF

You will then see the following:

*******************************************************

******************** PROFILE PROGRAM ******************

*******************************************************

No DINP file exists for PROFILE program.

Enter filename on the next line >>>>>

Using file: DINP.SAM

The program is now executing, using the sample input

datafile, DINP.SAM, and when finished, the last lines should be:

PROFILE completed normally

Number of profiles generated:  9

Output File:  TXTOUT

The output file, TXTOUT, which contains the data to be

plotted as  well as the plotting commands, can then be compared

with the sample case output, TXTOUT.SAM, in the SAMPLE

subdirectory of the diskette.

If the user has the companion PSGRAPH report (Harlan et al.,

1991a), and the PSGRAPH program is in the current directory, the

plots can be printed and viewed on the screen.

Type:  RUNPS and type a carriage return for all the choices

(i.e., use defaults).

4.  HOW TO USE PROFILE

4.1  Create an Input Data File

The input data file, usually called DINP, has the same form

as input files for the HARPO program.  A DINP can contain an

unlimited number of "runsets," each of which plots a profile.

The file can have any name, up to 80 characters (including the

path), but both the FINDMODL program and PROFILE are expecting a

file called DINP.  If the file DINP is not found in the local

directory, the user will be prompted for a file name.

Use the blank order forms for the input data in Appendices

C1, C2, and C3 when creating an input DINP file to be sure that

the necessary input data are complete and accurate.

Note that the input data forms contain only those W-array

elements necessary for the particular profile type (as indicated

in the input data forms) except for W(29), which should always be

1.0.

See Appendix C5 for a table of current and future profile

types.

A format that is unique to the PROFILE program is available

for plotting tabular data (or point plots).  The sample case

input data file, DINP.SAM, contains three examples of such a

plot.  This format allows the user to include arrays of data that

can be plotted as individual points.  See Appendix C4 for more

description and an example.

When deciding on maximum/minimum values in a profile

[specified by W(92),W(93)], the user may choose to have the

PROFILE program compute appropriate maximum and minimum values.

This is called  "auto-scaling" and is selected by setting the

maximum and minimum  to the same value (0.0 is a good choice) or

by setting the  associated tick mark interval [e.g., W(94)] to

0.0.  Also, the  program will perform auto-scaling if the user

inadvertently submits an input data file with a tick mark

interval whose absolute value is larger than both the maximum and

minimum values.

As in HARPO, W-array values that do not change need not be

repeated after the first run set.  For example, you do not have

to enter the latitude of every profile if all profiles in the

file have the same latitude.  You can enter only the first

profile's latitude.

See the HARPO/HARPA report (Sec. 2.4) for more format

specifications for the input files.

4.2  The Two Modes for Running PROFILE with RUNPROF.BAT

A DOS batch file, RUNPROF.BAT, controls the linking and

running of the PROFILE program.

MODE 1.   Linking before running.

Used whenever a model is changed in the DINP file

since last invoking RUNPROF.  Not necessary when

only data used by a model is changed.

Type:  RUNPROF L

Several lines of linker commands will appear on

the screen.

MODE 2.   Running without linking.

Used when no changes (or non-model changes) have

been made to the DINP file since last invoking

RUNPROF.

Type:     RUNPROF

IMPORTANT:

The PROFILE program will look for and use a file

called "DINP" or, if not found, the last input

data file that was linked.  The program will place

a message on the screen when it is using a file

other than "DINP".  See Section 4.2.1.

FOR BOTH MODES:

When the PROFILE program begins, the following lines

will appear:

*******************************************************

******************** PROFILE PROGRAM ******************

*******************************************************

The program is now executing and when finished, the last lines

should be:

PROFILE Completed Normally.

Number of profiles generated:

Output File:  TXTOUT.

Assuming the user has the companion report (Harlan et al.,

1991a), and that the program, PSGRAPH, from that report has been

loaded, the plots can now be viewed.

Type: RUNPS and type a carriage return for all the choices

(i.e., use defaults).

4.2.1  Using Input Data File other than DINP with RUNPROF.BAT

For mode 1 (linking before running) if there is no file

named DINP in the local  directory, the user is prompted by

FINDMODL for a file name as  follows:  (Note that bold lowercase

indicates user-selected input, bold uppercase indicates non-

selectable input.)

No DINP file exists for FINDMODL program.

Enter file name on the next line >>>>>

Type:  filename

When using a file with a name other than DINP, the user may

avoid being prompted by FINDMODL by typing:  RUNPROF L filename

Then, when the PROFILE program begins, the following will

appear. (The user does not have to re-enter the file name, since

the file name entered in response to the previous prompt is

used.)

No DINP file exists for PROFILE program.

Enter file name on the next line >>>>>

Using file:  filename

The RUNPROF.BAT file re-directs standard input from a file

called FNAME.OUT.  This file contains the name of the input data

file when it is something other than "DINP".  It was created by

FINDMODL and serves as a communications channel between the two

programs for the input file name.  It was created by FINDMODL and

serves as a communications channel between the two programs for

the input file name.  Therefore, be sure  not to delete this file

when using RUNPROF.BAT.

4.3  Compiling

It is not necessary to compile any of the source code

provided in the distribution disk unless you modify it.

The object modules and the source code for the model

subroutines are provided in the distribution disk subdirectory,

MODELS.

If modifications to PROFILE are desired, the COMPROF.BAT

batch file contains the compiler instructions to compile all the

PROFILE source code.

5.  HOW PROFILE WORKS

5.1  Basic Overview

The operation of PROFILE is complicated by the fact that the

input data file, DINP, indicates which model subroutines must be

linked with the main body of the PROFILE program.  The FINDMODL

program reads DINP to determine which models will be linked to

produce the executable file, PROFILE.EXE.

Creating profiles from an input file (DINP) is actually a

three-step process:

1.  Run FINDMODL

2.  Invoke the MicroSoft Linker

3.  Run PROFILE.

The DOS batch file, RUNPROF.BAT, performs all three steps

when you type "RUNPROF L" or "RUNPROF L filename".  When you type

"RUNPROF" with no parameter, it will bypass the FINDMODL program

and the linking process.  This is desirable if no changes have

been made to the model selections in the DINP file.

The first program, FINDMODL, reads the DINP file to

determine which models are to be used to construct the profile.

If there is no file named DINP in the local directory, the user

is prompted for a file name.  That file name is then written to a

file named FNAME.OUT (which is later used by the PROFILE program,

see below).

The primary function of FINDMODL is to create an output

file, called LINKMODL.DAT, which is used as input to the

MicroSoft Linker.  This file, called a "response file" by

MicroSoft, contains a list of the names (and locations) of all

object modules for the models to be linked.

Appendices D and E show the linker response file for the

sample case and the linker commands that result from the

response file.

Then, the linker is invoked, producing the executable file,

PROFILE.EXE, which is the PROFILE program linked with the models

selected by the user's DINP file.

The RUNPROF batch file redirects standard input to the

PROFILE program from FNAME.OUT.  Thus, when a file named DINP is

not available, the user is prompted for a file name but the

FNAME.OUT file provides the response to the prompt.

Then, PROFILE executes, producing the output text file,

TXTOUT.  TXTOUT contains plotting commands that can be used as

input to the DISSPLA plotting package or as input to the IBM

PC-compatible program PSGRAPH.

5.2  The DINP file and Linking the PROFILE Program

The PROFILE program and the models that are linked with it

are all written in machine-independent FORTRAN.

Because all the models for a particular parameter (e.g.,

background ocean current) have the same entry point names, it is

not possible to link all models when creating the executable

program.  Most linkers generate "re-definition" errors when faced

with this situation.  Therefore, it is necessary to link only

those models that have been selected via the input (DINP) data

file.

5.3  How FINDMODL Works

FINDMODL simply reads the input DINP file to determine all

models that are to be linked with the main PROFILE program.

Before reading the DINP file, however, FINDMODL reads the

"locations" file, MODELDIR.DAT (described in Section 4.1.2).  The

one line of text in MODELDIR.DAT is written to LINKMODL.DAT in

the appropriate places.

A character string table containing all the model names and

their associated check numbers is in the INCLUDE file,

MODLTABL.DAT.

The FINDMODL program simply compares the numbers found in

DINP with those in MODLTABL.DAT and extracts the appropriate

character string from the table; e.g., if W(150) in DINP is 6, it

indicates the CSMUNK2 sound speed model.

In order to satisfy all external references for the PROFILE

program, several "stub" subroutines have been created (e.g.,

NOSURF, NOCURR).  These are included in the LINKMODL.DAT file

when the DINP file does not have a selection for a particular

type of model (e.g., surface model, current model).

The user can modify MODLTABL.DAT to include any new models

created.  Of course, it would then be necessary to recompile and

relink FINDMODL.

6.  References

Georges, T.M., R. Michael Jones, and R.S. Lawrence, 1990, A PC

version of the HARPO ocean acoustic ray-tracing program,

NOAA Tech. Memo.  ERL WPL-780, NOAA Environmental Research

Laboratories, Boulder, Colorado, 18 pp. + disk.

Jones, R.M., J.P. Riley, and T.M. Georges, 1986a, HARPA--A

versatile three-dimensional Hamiltonian ray-tracing program

for acoustic waves in an atmosphere above irregular terrain,

NOAA Report, Environmental Research Laboratories, Boulder,

CO, 410 pp.

Jones, R.M., J.P. Riley, and T.M. Georges, 1986b, HARPO--A

versatile three-dimensional Hamiltonian ray-tracing program

for acoustic waves in an ocean with irregular bottom, NOAA

Report, Environmental Research Laboratories, Boulder, CO,

455 pp.

Harlan, J.A., R.M. Jones, and T.M. Georges, 1991a, PSGRAPH--A

program to generate PostScript files from PC-HARPO, PROFILE

and CONPLT, NOAA Tech. Memo. in preparation.

Harlan, J.A., T.M. Georges, and R.M. Jones, 1991b, CONPLT--A

program to generate contour plots from HARPO/HARPA input

files, NOAA Tech. Memo. in preparation.

Appendix A.  Instructions for Non-PC Users and PC-Users

Without MicroSoft Compiler/Linker Utilities

A.1.  General Guidelines

The PROFILE program is made up of scores of subroutines

extracted intact from HARPA/HARPO and contains approximately 2750

lines of code, not including the models.  Even though it is often

desirable to allocate a single module of a program to a unique

file, the large number of subroutines makes this impractical.

In addition, the use of multiple entry points within

subroutines inherently precludes the "one module-one file"

philosophy.

It is left to the user to decide whether or not to combine

PROFILE.FOR, PCPROFx.FOR and CONBLK.FOR into one larger file.

Obviously, object modules must be created for each of the

model subroutines using your own compiler.

Also, the batch files (*.BAT) provided in the distribution

disk have no direct use for non-MicroSoft systems.  However, they

may provide some guidance in designing equivalent functions,

whether via files or operating system commands.

The correct object modules for the models selected in the

DINP file must be linked with the main portion of PROFILE before

running.  Therefore, the FINDMODL program MUST BE MODIFIED in

order to create output equivalent to LINKMODL.DAT for your own

operating system.  See Section 5.3, the FINDMODL source code and

the appropriate section below corresponding to your operating

system for more information.

A.2.  Running the Sample Case

After FINDMODL has been modified and the correct model

object modules have been linked with the main program, PROFILE

can be run.  The output file, TXTOUT, can then be compared with

the sample case, TXTOUT.SAM, on the diskette.

A.3.  Instructions for CYBER NOS Users

A.3.1  Compilation Considerations for PROFILE

The following changes need to be made to the source code

furnished by the distribution disk.  See the Historical Note,

Appendix F.

1.   Throughout all files, replace "DOUBLE PRECISION" with

"REAL".

2.   Throughout all files, replace double precision intrinsic

functions with their single precision equivalents:

DMAX1 with AMAX1

DINT with AINT

DLOG10 with ALOG10

DABS with ABS

A.3.2  Source Code Changes Necessary to FINDMODL

Section 5.3 contains a description of FINDMODL's functions.

The FINDMODL output file, LINKMODL.DAT, will have to have a

different form.  The likely candidate is a NOS procedure file.

The file will need to specify the object modules of those models

which need to be linked with the main PROFILE program.

This may be done by explicitly specifying each object module

or by putting all the object modules in a library which is then

linked with the main program.

The reference to GETARG must also be removed.  Compile

FINDMODL after appropriate changes have been made.  The INCLUDE

file, MODLTABL.DAT, must be present during compilation.

A.4.  Instructions for VAX VMS Users

A.4.1  Compilation Considerations for PROFILE

In subroutine CONBLK, replace "1H" with "1H ".  That is,

make sure that there is a space after the H.  This is an oddity

of VAX and VAX editors.

A.4.2  Source Code Changes Necessary to FINDMODL

Section 5.3 contains a description of FINDMODL's functions.

The FINDMODL output file, LINKMODL.DAT will have to have a

different form.  The likely candidate is a DCL command file.  The

file will need to specify the object modules of those models

which need to be linked with the main PROFILE program.

This may be done by explicitly specifying each object module

or by putting all the object modules in a library which is then

linked with the main program, probably using a VMS Linker options

file.

The reference to GETARG must also be removed.  Compile

FINDMODL after the appropriate changes have been made.  The

INCLUDE file, MODLTABL.DAT, must be present during compilation.

A.5.  Instructions for UNIX Users

A.5.1  Compilation Considerations for PROFILE

Use the option, "-Nx250", to "f77" which increases the

maximum number of external names (e.g. COMMON blocks, function

names, entry points etc.) from 200 (default) to 250.

A.5.2  Source Code Changes Necessary to FINDMODL

Section 5.3 contains a description of FINDMODL's functions.

The FINDMODL output file, PROFLNKR.BAT, will have to have a

different form.  The likely candidate is a "makefile."  The file

will need to specify the object modules of those models which

need to be linked with the main PROFILE program.  This may be

done by explicitly specifying each object module or by putting

all the object modules in a library which is then linked with the

main program.

With UNIX, much of this could be done when invoking the

"f77" command.

The reference to GETARG must also be removed.  Compile

FINDMODL after the appropriate changes have been made.  The

INCLUDE file, MODLTABL.DAT, must be present during compilation.

A.6.  Instructions for PC Users without MS-Utilities

A.6.1  Compilation Considerations for PROFILE

The source code should be easily compiled although PROFILE

has not, as of this writing, been ported to a PC without

MS-Utilities.

A.6.2  Source Code Changes Necessary to FINDMODL

Section 5.3 contains a description of FINDMODL's functions.

The FINDMODL output file, LINKMODL.DAT, will have to have a

different form.  The likely candidate is a batch file.  The file

will need to specify the object modules of those models which

need to be linked with the main PROFILE program.

This may be done by explicitly specifying each object module

or by putting all the object modules in a library which is then

linked with the main program.

Compile FINDMODL after the appropriate changes have been

made.  The INCLUDE file, MODLTABL.DAT, must be present during

compilation.

Appendix B.  Description of the Distribution Disk Contents

B.1  Main Directory of the Distribution Disk

PROFILE.FOR

PCPROFx.FOR (x = 1,2,3,4,5,6)

CONBLK.FOR

All source code for PROFILE and its subroutines.

PROFILE was separated into 8 files due to its large

size.  This separation was somewhat arbitrary although

the intent was to have files that could be compiled

without exceeding the 64 Kbyte limit of the MicroSoft

compiler.

FINDMODL.FOR

Source code for FINDMODL, which reads the input data

file (usually DINP) to determine which models will be

linked with the main PROFILE program.

MODLTABL.DAT - An INCLUDE file for FINDMODL.

COMPROF.BAT - use for compiling all source code in the main

directory of the distribution diskette.

RUNPROF.BAT - use for linking and running PROFILE.

FINDMODL.EXE - Executable version of FINDMODL.

PROFILE.EXE - Executable version of PROFILE for the sample

case.

FNAME.OUT

Data File for communicating the input data file name from

FINDMODL to PROFILE program.  On distribution diskette,

contains "DINP.SAM".

DINP.SAM - Input data file for the sample case.

LINKMODL.SAM - output from FINDMODL for the sample case.

TXTOUT.SAM - output from PROFILE for the sample case.

INSTALL.BAT - Batch file which creates models subdirectory

and copies diskette contents to user's

machine.

INSTALIT.EXE - Executable file called by INSTALL.BAT.

PROFILE.DOC - The report you are reading.

B.2  Subdirectories of the Distribution Disk.

1.   MODELS Subdirectory.

Contains source code and object modules for all the

models.

The object modules are provided for MS-Utilities users.

There is no need to compile the model source code if

you are using MS-Utilities.

Note that older versions of PC-HARPO models are not

compatible with PROFILE due to the following two

changes:

a.   New error handling has eliminated the use of the

RERROR routine.

b.   Changes to the WW common block.

Appendix C.  Blank Input Data Forms

There are currently three different types of profiles that

can be generated by the PROFILE program:

1.   Sound Speed/Current Speed/Wind Speed

2.   Temperature

3.   Topography

In addition, PROFILE will generate plots of data points

entered by the user in the input data file.  See APPENDIX C4.

Appendix C1.  Form to Specify Input Data for Profiles

of Sound Speed/Current Speed/Wind Speed

Model ID:__________

Profile type:  sound speed __________(W90=1.0)

current/wind speed __________(W90=2.0)

vertical component of current/wind velocity __________(W90=3.0)

southward component of current/wind velocity __________(W90=4.0)

eastward component of current/wind velocity __________(W90=5.0)

Superimpose this profile on the graph of the previous runset:

Yes __ (W90 negative; suppresses frame advance before plotting.)

No  __ (W90 positive.)

Latitude of the profile = __________ (rad, deg, km) north (W83)

Longitude of the profile = __________ (rad, deg, km) east (W84)

Height above sea level of bottom of graph = __________ km (W88)

Height above sea level of top of graph = __________ km (W89)

Minimum speed = __________ km/s, m/s (W92) (auto-scaling if zero)

Maximum speed = __________ km/s, m/s (W93) (auto-scaling if zero)

Horizontal tick mark interval = __________ km/s, m/s (W94)

(auto-scaling if zero)

Vertical tick mark interval = __________ km (W96)

(auto-scaling if zero and defaults to meters)

Appendix C2.  Form to Specify Input Data for

Profiles of Topography

Model ID:__________

Profile type:                      topography __________(W90=6.0)

Superimpose this profile on the graph of the previous runset?

Yes __ (W90 negative; suppresses frame advance before plotting.)

No __ (W90 positive.)

Left Latitude of the profile = _________ (rad, deg, km) north

(W83)

Longitude of the profile = _________ (rad, deg, km) east (W84)

Right Latitude of the profile = ________ (rad, deg, km) north

(W85)

Appendix C3.  Form to Specify Input Data for

Profiles of Temperature

Model ID:__________

Profile type:                      temperature

__________(W90=7.0)

Superimpose this profile on the graph of the previous runset?

Yes __ (W90 negative; suppresses frame advance before plotting.)

No ___ (W90 positive.)

Latitude of the profile = __________ (rad, deg, km) north (W83)

Longitude of the profile = __________ (rad, deg, km) east (W84)

Height above sea level of bottom of graph = __________ km (W88)

Height above sea level of top of graph = __________ km (W89)

Minimum Temperature (auto-scaling if zero) = ________ deg K (W92)

Maximum Temperature (auto-scaling if zero) = ________ deg K (W93)

Horizontal Tick Mark Interval (auto-scaling if zero) = _________

deg K (W94)

Vertical tick mark interval = __________ km (W96)

(auto-scaling if zero and defaults to meters)

Appendix C4.  Format for Tabular Data Input

The tabular data must always have the same general format as

the example given below:

0T6    XYZ DATA POINTS

90-45.0000000000   NEGATIVE VALUE = SUPERIMPOSE ON PREVIOUS PLOT

-45                 NEGATIVE VALUE = TABULAR DATA SET FOLLOWS

A                 Descriptive Comments about Data Set

2      999.0

LN KM         LN KM

1.06103       295.1175

1.04327       295.1062

1.00858       295.0950

1.10932       295.0837

1.20206       295.0725

1.38054       296.0613

1.28684       296.0501

1.40138       296.0390

1.47047       296.0278

999.000

0                      RETURNING TO W ARRAY DATA SET

0

The following lines must be present:

1.   Model ID and Title Line

2.   90 followed by a (usually negative) value from 41 to 62.

3.   A negative value which is the same as that given in line 2.

4.   Any descriptive comments (optional)

5.   2 followed by a flag value which indicates the end of the

data when it is placed in the first column of numbers.

6.   Indications of the units for the data set (optional) for

each column following the format requirements given in HARPO

report.

7.   2 columns of input data of 13 columns each of any floating

point format.

8.   Flag value in first column.

9.   0 optionally followed by some commentary starting in column

25.

10.  0 indicating the end of the runset.

Appendix C5.  Table of W(90) Values to Specify Profile Type

W(90) Value         Common Block        Profile Type

1                   CC             Sound Speed

2                   UU             Current/Wind Speed

3                   UU             Vertical Component

4                   UU             Southward Component

5                   UU             Eastward Component

6                   GG             Topography

7                   TT             Temperature

8*                  MM             Molecular Weight

9*                  AA             Viscosity

10*                 AA             Thermal Conductivity

11*                 LL             Absorption

12*                 PP             Pressure

>40  and <63                            Point Plots

* - indicates not yet implemented but values have been

reserved.

Appendix D.  The Linker Response File for the Sample Case

LINKMODL.DAT

models\VVORTX3+

models\NPCURR +

models\CSMUNK1+

models\NPSPEED +

models\TLINEAR +

models\NPTEMP +

models\RHORIZ +

models\GLORENZ +

models\NPBOTM +

models\NOLOSS +

models\NOSURF

This file provides input to the LINK command.  See the MicroSoft

Linker manual for explanations of the syntax.

Appendix E.  Linker Instructions Generated

for the Sample Case

MicroSoft (R) Segmented-Executable Linker  Version 5.03

Copyright (C) Microsoft Corp 1984-1989.  All rights reserved.

Object Modules [.OBJ]: /STACK:4096 /SEGMENT:130 /NOE  MODELS\VVORTX3+

Object Modules [.OBJ]:  MODELS\NPCURR +

Object Modules [.OBJ]:  MODELS\CSMUNK1+

Object Modules [.OBJ]:  MODELS\NPSPEED +

Object Modules [.OBJ]:  MODELS\TLINEAR+

Object Modules [.OBJ]:  MODELS\NPTEMP +

Object Modules [.OBJ]:  MODELS\RHORIZ +

Object Modules [.OBJ]:  MODELS\GLORENZ+

Object Modules [.OBJ]:  MODELS\NPBOTM +

Object Modules [.OBJ]:  MODELS\NOSURF +

Object Modules [.OBJ]:  MODELS\NOLOSS

Run File [PROFILE.EXE]:

Appendix F.  Historical Note on the Use of Double Precision

The PROFILE program was originally written on a CYBER machine

using numerous common blocks and Hollerith data instead of character

data.  HARPO and PROFILE use the same common blocks and many of the

same routines.  While updating the PROFILE program for ASCII text

output, a principal design goal was to maintain as much compatibility

with HARPO as possible.  Because of the unusual (60-bit) word length

of the CYBER and the CYBER's 6-bit characters, 10 characters could be

put in a single word.  This fact, coupled with our design goal, caused

us to use double precision variables to accommodate more characters

per word.
      Appendix G.  PROFILE Plotting Commands

G.1  Plotting Commands in TXTOUT Used by PSGRAPH

22   Lower left origin (in inches from corner)

25   Title Text and Axis Labels

29   Points to be plotted

30   Frame advance

31   End of plotting

39   Coordinates for plot (in plotting units)

G.2  Plotting Commands in TXTOUT Ignored by PSGRAPH

10   Font indicator

11   Case indicator

12   Case indicator

13   Text Height

20   Initializes DISSPLA to create an

intermediate compressed output file

21   Clipping Indicator

23   Page Limits

24   Scale Factor for plot symbols

26   Draw a frame

32   Number of X axis ticks

33   Number of Y axis ticks

35   For converting label values

36   Suppress border

G.3  FORTRAN Format Specification for Plotting Commands

Command

10   Font indicator

Number of Lines:  1

Contents of Line 1:  "10"

Format of Line 1:  I2

11   Case indicator

Number of Lines:  1

Contents of Line 1:  "11 STAND     !"

Format of Line 1:  I2,trl,a,lx,al

12   Case indicator

Number of Lines:  1

Contents of Line 1:  "12 L/CSTD    #"

Format of Line 1:  I2,trl,a,lx,al

13   Text Height

Number of Lines:  1

Contents of Line 1:  "13" number

Format of Line 1:  I2,F4.2

20   Initializes DISSPLA to create an intermediate compressed output

file

Number of Lines:  1

Contents of Line 1:  "20"

Format of Line 1:  I2

21   Clipping Indicator

Number of Lines:  1

Contents of Line 1:  "21" number

Format of Line 1:  I2,F4.2

22   Lower Left Origin (in inches from lower left corner)

Number of lines:  1

Contents of Line 1:  "22" X Y

Format of Line 1:  I2 2(F5.2)

23   Page Limits

Number of Lines:  1

Contents of Line 1:  "23" number, number

Format of Line 1:  I2,F6.2

24   Scale Factor for plot symbols

Number of Lines:  1

Contents of Line 1:  "24" number

Format of Line 1:  I2,F5.2

25 - Text for Title and Axis Labels

Number of Lines:  4

Contents of Line 1:  "25" X-axis length, Y-axis length (inches)

Contents of Line 2:  Title length, title

Contents of Line 3:  X-axis label length, X-axis label

Contents of Line 4:  Y-axis label length, Y-axis label

Format of Line 1:  I2 2(F6.2)

Format of Line 2:  I5 1X A40

Format of Line 3:  I5 1X A40

Format of Line 4:  I5 1X A40

29 - Points to be Plotted for a Curve

Number of Lines:  Variable

Contents of Line 1:  "29", Number of points, Symbol Marker

Contents of Line 2 to N:  x(i), y(i), x(i+1), y(i+1)

where i = 1 to Number of points

Format of Line 1:  I2, 2(I5)

Format of Line 2 to N:  4(E14.6)

NOTE:  If the number of points is odd, then the last line has

only one x,y pair.  Thus, the Nth line would have format

2(E14.6).

30 - Frame Advance for Profiles

Number of Lines:  1

Contents of Line 1:  "30  1"

Format of Line 1:  I2, I3

31 - End of Plotting

Number of Lines:  1

Contents of Line 1:  "31"

Format of Line 1:  I2

32   Number of X axis ticks

Number of Lines:  1

Contents of Line 1:  "32" number

Format of Line 1:  I2,I3

33   Number of Y axis ticks

Number of Lines:  1

Contents of Line 1:  "33" number

Format of Line 1:  I2,I3

35   For converting negatively valued labels to positive numbers.

Begin converting at the "mth" number and continuing for "n"

numbers.

Number of Lines:  1

Contents of Lines 1:  "35" m, n

Format of Line 1:  I2, 2I3

36   Suppress border

Number of Lines:  1

Contents of Line 1:  "36"

Format of Line 1:  I2

39   Maximum, minimum and interval values for both axes in real units.

Number of Lines:  1

Contents of Line 1:  "39",

X-axis minimum, X-axis interval, X-axis

Y-axis minimum, Y-axis interval, Y-axis

Format of Line 1:  I2, 6(E13.7)