NOAA Technical Memorandum for ERL WPL-
203

PSGRAPH - A Plotting Program for PC-HARPO, PROFILE,

CONPLT, and EIGEN

J.A. Harlan

R.Michael Jones

T.M. Georges

NOAA/Wave Propagation Laboratory

NOTICE

Mention of a commercial company or product does not constitute an

endorsement by NOAA/ERL.  Use for publicity or advertising purposes,

of information from this publication concerning proprietary products or

the tests of such products, is not authorized.  
                            CONTENTS

PAGE

1.  INTRODUCTION

1.1  What PSGRAPH Does

1.2  For Non-PC Users

1.3  What PSGRAPH Does Not Do

2.  INSTALLATION

2.1  Installing PSGRAPH from the Distribution Disk

2.2  Installing PSGRAPH via Internet

3.  SAMPLE CASES

3.1  Sample Case 1:  Profile Plots

3.2  Sample Case 2:  PC-HARPO Ray Plots

3.3  Sample Case 3:  Contour Plots

3.4  Sample Case 4:  Plots from EIGEN

3.5  Sample Case 5:  Eigenray Plot from PC-HARPO

4.  HOW TO USE PSGRAPH

4.1  Input and Output

4.2  The Two Modes for Running PSGRAPH

4.2.1  Interactive Mode

4.2.1.1  User Responses to Prompts

4.2.1.1.1  Input File Name

4.2.1.1.2  PostScript Output File Name

4.2.1.1.3  Advanced Features

4.2.1.1.3.1  PostScript Scale Factor

4.2.1.1.3.2  PostScript Font Size

4.2.1.1.3.3  PostScript Line Width

4.2.1.1.3.4  Ray-editing Factor for PostScript Files

4.2.1.1.3.5  Slow Motion Factor

4.2.2  Standard Input Re-Direction Mode

4.3  Troubleshooting

4.3.1  CLRS - Resetting the Terminal to Text Mode

4.3.2  Integer Overflow Errors

4.3.3  READ Errors while Reading TXTOUT

4.3.4  SHOWMODE - Determining Your Monitor's Video Mode

5.  HOW PSGRAPH WORKS

5.1  Basic Overview

5.2  Program File Structure

5.3  Program Functional Structure

5.4  Some Features of PSGRAPH

6.  References

Appendix A.  Source Code Changes to PC-HARPO Necessary for PSGRAPH Compatibility

Appendix B.  Description of the Distribution Disk

Appendix C1. Input Data File Listings for Sample Case 1

Appendix C2. Input Data File Listings for Sample Case 2

Appendix C3. Input Data File Listings for Sample Case 3

Appendix C4. Input Data File Listings for Sample Case 4

Appendix C5. Input Data File Listings for Sample Case 5

Appendix D.  FORTRAN Format Specifications for Plotting Commands

Part I.  Commands Used by PSGRAPH

Part II. Commands Used by DISSPLA (Ignored by PSGRAPH)

List of Figures

1. Profile Plot - Sound Speed from Sample Case 1.

2. Profile Plot - Current Speed from Sample Case 1.

3. Profile Plot - Southward Current Component from Sample Case 1.

4. Profile Plot - Eastward Current Component from Sample Case 1.

5. Profile Plot - Temperature from Sample Case 1.

6. Profile Plot - Bottom Topography from Sample Case 1.

7. Ray Plot - Vertical Plane, Rectangular Plot from Sample Case 2.

8. Ray Plot - Horizontal Plane from Sample Case 2.

9. Ray Plot - Vertical Plane, Polar Plot, Radial Expansion from Sample Case 2.

10. Ray Plot - Vertical Plane, Rectangular Plot from Sample Case 2.

11. Contour Plot - Sound Speed, Vertical Plane, Rectangular Plot from Sample Case 3.

12. Contour Plot - Current Speed, Vertical Plane, Rectangular Plot from Sample Case 3.

13. Contour Plot - Sound Speed, Vertical Plans, Polar Plot, Radial Expansion from Sample Case 3.

14. Contour Plot - Current Speed, Horizontal Plane, Lateral Expansion   from Sample Case 3.

15. Contour Plot - Southward Current Component, Vertical Plane, Rectangular Plot from Sample

Case 3.

16. Contour Plot - Bottom Topography, Horizontal Plane, Lateral Expansion from Sample Case 3.

17. Contour Plot - Eastward Current Component, Vertical Plane, Rectangular Plot from Sample Case

3.

18. Eigen Plot - Range vs. Elevation Angle plot from Sample Case 4.

19. Eigen Plot - Range vs. Time Delay plot from Sample Case 4.

20. Eigenray Plot - Eigenrays from Sample Case 5.
PSGRAPH - A Plotting Program for PC-HARPO,

PROFILE, CONPLT and EIGEN.

J.A. Harlan, R.M. Jones and T.M. Georges

ABSTRACT.  We describe a FORTRAN computer program,

PSGRAPH, that generates PostScript graphics files and PC-compatible

screen graphics.  Input for the program is provided by files produced by

the PROFILE, CONPLT, and EIGEN programs and the PC version of

the HARPO program.  The program provides for user-selectable

PostScript plotting parameters including text font size, magnification

factor and line width of the plots.  In addition, a data editing factor

allows the user to reduce the size of the output PostScript file while

retaining the resolution of the plot.  Included is a floppy disk with

computer programs and sample cases.  Instructions for retrieving

PSGRAPH via Internet are also given.

1.  INTRODUCTION

1.1.  What PSGRAPH Does

The PC version of HARPO (Georges et al., 1990), PROFILE (Harlan et al., 1991a),

EIGEN (Weickmann et al., 1987), and CONPLT (Harlan et al., 1991b) programs produce

ASCII text files which contain plotting commands.  These commands are interpreted by

PSGRAPH to produce PostScript plots as well as screen graphics using MicroSoft Graphics

Library commands.

PSGRAPH is specifically designed to process the outputs of only PC-HARPO, PROFILE,

CONPLT and EIGEN.  The PSGRAPH program's primary function is to create publication

quality plots using PostScript graphics.  An additional function is to simultaneously produce

screen graphics on PC-compatible video monitors.  However, the screen graphics are designed

to provide a quick preview of the PostScript graphics and thus are not as detailed.

NOTE:  When referring to PC-HARPO, this report refers to PC-HARPO versions dated

December 1990, or later.  Included in Appendix A is a listing of the source code changes for

subroutines in PC-HARPO which will update older PC-HARPO versions to be compatible

with PSGRAPH.

1.2.  For Non-PC Users

The PSGRAPH program can be used by non-PC users after some modification.  The

program is designed so that the PostScript graphics modules are separate from the screen

graphics modules. Thus, you can either

1)  replace the screen graphics modules with ones that would work on your operating

system and display monitor, or

2)  remove all the screen graphics modules and use only the PostScript output files.  While

this option might, at first, seem inadequate, there is commercially available software

which makes it possible to display standard PostScript files on a video monitor.  Thus

the PostScript file could also be used to produce the screen graphics output.

1.3.  What PSGRAPH Does Not Do

The program is not compatible with any input data other than that produced by PROFILE,

PC-HARPO, EIGEN and CONPLT.  In particular, as noted above, it is not compatible with

PC-HARPO version 7-1-90.

Also, the program will not process TXTOUT files that are combined by the user from

different programs, e.g., the output from PC-HARPO and PROFILE combined into a single

TXTOUT file.

2.  INSTALLATION

2.1.  Installing PSGRAPH via the Distribution Disk

Copy the entire contents of the distribution disk to your working directory.  The

distribution floppy disk (1.2 MB, 5-1/4" IBM-PC/AT format) contains ASCII files that

include FORTRAN source code, batch files, the input and the output files for the sample

cases.  Also included on the disk is the executable file, PSGRAPH.EXE, and a copy of the

document you are reading.

A complete description of the contents of the disk is given in Appendix B.

2.2.  Installing PSGRAPH via Internet

The PSGRAPH program and its peripheral programs (i.e., the contents of the distribution

disk) can be obtained via Internet and anonymous FTP.  The "anonymous" account contains a

public directory, "pub", which contains a subdirectory "raytracing", which is divided into

several more subdirectories.  The subdirectory to "raytracing" called "psgraph" contains the

files you will need.  To access these files via anonymous FTP through the Internet, proceed

as follows:

1. Log on to a host at your site that has an Internet connection and also supports the FTP

command.

2. Invoke FTP by entering the Internet address of the server at the NOAA Wave Propagation

Laboratory site:

ftp pooh.wpl.erl.gov

or

ftp 140.172.32.11

3. Log in as "anonymous"

4. For the password, enter your complete Internet address, e.g., jah@node.dee.dum.gov

5. General instructions and information about the anonymous FTP account at our site is

available in a file named "README" in the "pub" subdirectory.

To transfer this file to your local host, first change to the "pub" directory:

cd pub

and then enter:

get README     (Note that the name of the file is in uppercase.)

6. Change your current directory to the "raytracing/psgraph" subdirectory by entering:

cd raytracing/psgraph

7. To transfer information about the psgraph subdirectory to your local host, enter:

get readme.psg

This file also contains information about updates/corrections to PSGRAPH.

8. To transfer those files in the psgraph subdirectory that you are interested in, enter:

get filename

where filename is the name of a file that you want to transfer.  You may also retrieve

multiple files using "mget".

9. To return to your local host, enter:

quit

3.  SAMPLE CASES

The sample cases were produced by the PROFILE, PC-HARPO, CONPLT and EIGEN

programs.  The data files that were input to these programs are listed in Appendixes C1

through C5.

The distribution disk contains the data file which is input to PSGRAPH for each sample

case, named "TXTOUTx".  Also,included are the associated output PostScript files for sample

case 1 and sample case 2.

3.1.  Sample Case 1:  Profile Plots

Sample case 1 for PSGRAPH is the same as that used by the companion report (Harlan et

al.,1991a) for the PROFILE program.  The input file is TXTOUT.

This sample results in six separate PostScript plots with the first plot having two sets of

discrete data points superimposed on the plot curve.  The fifth plot has a single set of data

points superimposed on it.  The PostScript plots are shown in Figs. 1 through 6.

Run the sample case as follows:

Type:  RUNPS

You will be prompted for several input parameters.  Type a carriage return for each (i.e.,

use the defaults).

The output PostScript file, PS.OUT, can then be sent to a PostScript printer.  It should be

identical to PS.SAM from the distribution disk.

3.2.  Sample Case 2:  PC-HARPO Ray Plots

Sample case 2 provides four rayplot examples.  The input file is TXTOUT2.  It was

produced by the PC version of HARPO.

Type:  RUNPS

For Input File Name,  enter: TXTOUT2.

Enter defaults for all the other parameters.

The output file, PS.OUT, should be identical to PS2.SAM from the distribution disk.

The plots produced by the PostScript file are shown in Figs. 7 through 10.

3.3.  Sample Case 3:  Contour Plots

Sample case 3 provides seven contour plots.  The input file is TXTOUT3.  It was

produced by the CONPLT program.  This is the same sample case as that used by the

companion report (Harlan et al., 1991b)

Type:  RUNPS

For Input File Name,  enter: TXTOUT3

Enter defaults for all the other parameters.

The plots produced by the PostScript file are shown in Figs. 11 through 17.

3.4. Sample Case 4:  Plots from EIGEN

Sample Case 4 provides 2 plots.  The two plots were produced by the EIGEN program.

The input file is TXTOUT4.

Type:  RUNPS

For Input File Name,  enter: TXTOUT4

Enter defaults for all the other parameters.

The plots produced by the PostScript file are shown in Figs. 18 and 19.

3.5. Sample Case 5:  Eigenray Plot from PC-HARPO

Sample Case 5 provides 1 plot.  The plot was produced by the PC-HARPO program.  The

input file is for PSGRAPH is TXTOUT5.   (The input to PC-HARPO was created by the

EIGEN program for sample case 4.)

Type:  RUNPS

For Input File Name,  enter: TXTOUT5

Enter defaults for all the other parameters.

The plot produced by the PostScript file is shown in Fig. 20.

4.  HOW TO USE PSGRAPH

4.1.  Input and Output

There is only one input file to PSGRAPH:  an ASCII file created by PROFILE,

PC-HARPO, CONPLT or EIGEN programs.  There is also interactive input, discussed in

Section 4.2.1.

The output from PSGRAPH are a PostScript file and the screen graphics.

4.2.  The Two Modes for Running PSGRAPH

The batch file RUNPS.BAT can be used to run PSGRAPH interactively or using

redirection of standard input from a user-selected file that contains responses to the prompts

for input parameters.

4.2.1.  Interactive Mode

The interactive mode is the default mode.  To use this mode, either:

Type:  PSGRAPH

or

Type:  RUNPS

You are then prompted for each input parameter.

4.2.1.1.  Interactive Mode:  User Responses to Prompts

4.2.1.1.1. Input File Name

The input file name can be any valid DOS file name with or without path (up to 80

characters).

Default:  TXTOUT

4.2.1.1.2. Output PostScript File Name

The output PostScript file name can be any valid DOS file name with or without path (up

to 80 characters).

Default:  PS.OUT

4.2.1.1.3. Advanced Features

Type a "y" or "Y" to see the advanced features choices.  A new screen will appear which

will enable you to choose your own PostScript scale factor, PostScript font size, PostScript

line width, rayplot-editing factor and slow motion factor.  If the default, "n" or "N", is

chosen, PSGRAPH will use the defaults for all of the following plotting features.

Default:  N  (= Use defaults for the advanced features)

4.2.1.1.3.1. PostScript Scale Factor

The PostScript scale factor controls both the horizontal and vertical scales in the PostScript

output file.  If the scale factor is less than 1, the resulting plot is smaller than the default.  If

it is greater than 1, the plot is larger.  For 8.5" x 11" paper, a factor of 1 results in a plot that

uses about three-quarters of the page.

Default:  1.0

4.2.1.1.3.2. PostScript Font Size

The PostScript font size controls the size of the text for the axes labels.  The font for the

title of the plot is 1.5 times larger than the input font size.  The font type, which is not

user-selectable, is Helvetica.

Default:  12

4.2.1.1.3.3. PostScript Line Width

The PostScript line width is entered in inches.  It controls the width of the lines (curves)

drawn inside the frame.  The frame is always drawn with a line width of .01 inches.  The

frame line width is not user-selectable.

Default:  .01

4.2.1.1.3.4. Ray-Editing Factor for PostScript Files

When plotting output from PC-HARPO, this parameter allows you to reduce the size of

the PostScript file and reduce the execution time of PSGRAPH by reducing the number of

points that are written to the output PostScript file.  This can be done without loss of

resolution in the plot because the ray plot resolution is much higher than that of PostScript for

an 8.5" x 11" page.  The units of this parameter are HARPO plotting units, which divide the

plotting area into 10240 by 10240.  Essentially, without any editing factor, there would be

line segment "draw" commands that would result in lines smaller than would be resolved by

PostScript.  Since the PostScript files can be very large when no editing is done, e.g.,

hundreds of Kbytes, editing provides a means of reducing the size of the file without loss of

information.  An added bonus is that execution time of PSGRAPH is decreased because of

the decrease in WRITEs to the disk.

If the size of the PostScript file is of primary concern, the editing factor can be increased

to the maximum.  There may be an acceptable loss in resolution.  This must be tested on a

plot-by-plot basis.

The editing factor does not apply to profiles, contour plots or plots produced by the

EIGEN program.

Default:  10.0

4.2.1.1.3.5. Slow Motion Factor

The slow motion factor enables the user to slow the plotting on the screen for raypath

plots, that is, plots produced by PC-HARPO.  The larger the value selected, the slower the

plotting will be.  This feature can be especially helpful when viewing vertical projections

which are transverse to the raypath e.g., Fig. 10.

Default:  1.0  (no slow motion)

4.2.2.  Standard Input Redirection Mode

The standard input redirection mode uses the RUNPS batch file to redirect standard input

from the keyboard to a file chosen by the user.  This frees the user from having to enter

responses from the keyboard.  The file must consist of at least 3 lines of text, one per each

PSGRAPH prompt on the main screen.  If advanced features other than the defaults are to be

used, then at least 8 lines are necessary.

In addition, there must be a response for each plot to be displayed on the screen.  The

response consists of an "n" or a "y":  "y" instructs the program to pause after the plot, "n"

indicates no pause.  These responses are necessary because the program is waiting for user

response after each plot is made.  If a pause has been directed, the program will pause for 20

seconds after the plot.  A timer is visible in the upper right corner of the screen plot.  See the

file REDIRECT.SAM for a sample file of responses.  (NOTE:  The REDIRECT.SAM file

could be used to run the sample case using redirected input.)  This provides the user with a

means to produce PostScript plots in a "mass production" manner.  Also, this method may be

useful when the screen graphics are not of interest.

To use this mode,

Type:  RUNPS fname

where fname is the file containing the responses to the prompts (e.g., REDIRECT.SAM).

TIP FOR REDIRECTED INPUT FILES:  If you are unsure of the number of plots that

will be created from the input data, simply enter many more rows of "n" or "y" than you

think there are plots. There is no problem if there are too many responses, but too few

responses will result in a FORTRAN run-time READ error.

4.3.  Troubleshooting

4.3.1. CLRS - Resetting the Terminal to Text Mode

If a run-time error occurs while running PSGRAPH or if the user interrupts the program

via CTRL-C, the user must reset the terminal to text mode.  An executable file, CLRS.EXE,

is provided to do this.  Type:

CLRS

4.3.2.  Integer Overflow Errors

Most of the screen graphics use integer*2 variables.  If the user encounters an integer

overflow error, it usually indicates that there are one or more invalid values in the input

(TXTOUT) file.

These values would typically be one of the maximum, minimum or interval values for a

particular plot.  Check the input to the program e.g., PROFILE, which produced the TXTOUT

file and the TXTOUT file itself for any invalid values.

4.3.3.  READ Errors while Reading TXTOUT

These errors can occur if the TXTOUT file contains one or more invalid values.  Look at

the TXTOUT file for  asterisks in place of a value, which indicates that the value's magnitude

exceeded the FORTRAN format given to it.

4.3.4.  SHOWMODE - Determining Your Monitor's Video Mode

This program is provided to help the user troubleshoot problems with video setup.  Enter:

SHOWMODE and compare the values returned from the program with the values given in

the MicroSoft FGRAPH.FD file.  Each value indicates a particular type of video monitor e.g.,

16 = EGA 16 color with 640 by 350 pixels.

5.  HOW PSGRAPH WORKS

5.1.  Basic Overview

The input file to the PSGRAPH program must have been created by PROFILE,

PC-HARPO, EIGEN, or CONPLT.  PSGRAPH reads the file and interprets each line as either

a plotting command or as data associated with a plotting command.  Then, an appropriate

PostScript command is written to the output PostScript file and an appropriate MicroSoft

graphics library function/subroutine is called.

5.2.  Program File Structure

The source code has been divided into two files: PSGRAPH1.FOR and PSGRAPH2.FOR

PSGRAPH1.FOR contains the main program module and all the subroutines that write to

the PostScript file.

PSGRAPH2.FOR contains all the subroutines with Microsoft Graphics Library references.

This structure allows users to replace the MicroSoft Graphics Library references with their

own graphics routines or to eliminate the screen graphics entirely.

5.3.  Program Functional Structure

In addition to the division between subroutines for PostScript plotting and screen plotting,

there is a division between subroutines that perform functions for PROFILE input and those

that are for PC-HARPO input.  This division was necessary because of the very different

structure of the two types of input.  (CONPLT is similar in most ways to PC-HARPO but has

important differences.  Likewise, EIGEN is closely related to PROFILE but also has distinct

features of its own.)   The difference is primarily due to the use of different plotting

commands.  For the user who is interested in these command details, the source files contain

explicit comments about all commands.

Also, the screen graphics routines have been divided into very specific functional modules,

e.g., clearing the screen, labelling axes etc.  This will make it easier for the user to convert to

another graphics system.

Although the files produced by PROFILE/PC-HARPO/CONPLT/EIGEN (e.g., TXTOUT)

contain a combination of DISSPLA plotting commands and "meta-plotting-commands"

(Tables 1 and 2), many of the DISSPLA commands are not applicable or are unnecessary for

PostScript.  These commands are ignored by PSGRAPH (see Table 2).  The "meta-

commands" were created specifically for use by PSGRAPH.  Appendix D contains the

FORTRAN format specifications for all the plotting commands produced by the four

raytracing programs mentioned above.

5.4.  Some Features of PSGRAPH

5.4.1. Color Contour Lines

PSGRAPH will determine whether the user's video monitor has color capability.  If it

does, each contour level will be drawn with a separate color, starting with blue for the lowest

contour level.  The program will continue changing the color for each level until all the

available colors have been used, at which time it will return to blue and cycle through the

colors again.  Color contour lines are not currently implemented for the PostScript file.  Note

that for a typical 16-color monitor, only 14 colors are used; black and light white are not

used.

5.4.2.  Multiple Data Sets Superimposed on Plots

If the input data file to a program (e.g., PROFILE) contains tabular data, the points in the

tabular data are represented by a small circle centered about the point.  If there is more than

one tabular data set, they are represented by symbols in the following order:

1.  Small circle

2.  Small square

3.  Large circle

4.  Large square

If the user requires more than four data sets on a single plot, the PSGRAPH program can

be easily modified.  See subroutine INIT_PS_FILE in the source code file PSGRAPH1.FOR.

5.4.3.  Noninteger Intervals between Tick Marks

This feature provides the ability to plot an axis with different intervals between tick marks.

For example, the user may choose, in the input file to the PROFILE program, a depth range

of -3.0 km to +0.1 km.

5.4.4.  Auto-Varying Formats for Axis Labels

The maximum, minimum and increments for an axis can vary widely from plot to plot.

For example, sound speeds may typically be on the order of 1 km/sec whereas current speeds

may be on the order of 10-5 km/sec.  PSGRAPH will change the label format to handle the

entire range of an axis depending on the minimum, maximum, and increment values.

6.   References

Georges, T.M., R.M. Jones, and R.S. Lawrence, 1990, A PC version of the HARPO ocean

acoustic ray-tracing program, NOAA Tech. Memo.  ERL WPL-780, NOAA

Environmental Research Laboratories, Boulder, Colorado, 18 pp. + disk.

Harlan, J.A., T.M. Georges, and R.M. Jones, 1991a, PROFILE--A program to generate

profiles from HARPO/HARPA environmental models, NOAA Tech. Memo. ERL

WPL-198, NOAA Environmental Research Laboratories, Boulder, Colorado, 46 pp. +

disk.

Harlan, J.A., R.M. Jones, and T.M. Georges, 1991b, CONPLT--A program to generate

contour maps from HARPO/HARPA environmental models, NOAA Tech. Memo. in

preparation.

Weickmann, A.M., J.P. Riley, T.M. Georges, and R.M. Jones, 1989, EIGEN--A program to

compute eigenrays from HARPA/HARPO raysets, NOAA Tech. Memo. ERL WPL-

160, NOAA Environmental Research Laboratories, Boulder, Colorado, 91 pp.

Appendix A.  Source Code Changes to PC-HARPO Necessary for

PSGRAPH Compatibility

Note: Because of the extensive changes to Subroutine Plot, the entire

subroutine is listed.  The lower case code is new.

C**********************************************************************

SUBROUTINE PLOT (X,Y,NEW)

C**********************************************************************

C     PLOTS ONE VECTOR FROM CURRENT PLOT POSITION TO POINT(X,Y)

C     TAKING BORDER CROSSINGS INTO ACCOUNT.

C%    NEW = 0 means draw a line to X,Y;

C%    NEW .ne. 0 means move to X,Y without drawing a line.

implicit double precision (a-h,o-z)

COMMON /PLT/ XMIN0,XMAX0,YMIN0,YMAX0,RESET

COMMON/PLT/RMIN,RMAX,ALPHA,APLT

INTEGER REZFLG

PARAMETER (REZFLG=2001)

COMMON/DDREZ/DDHIX,DDHIY

COMMON/RAYDEV/NRYIND,NDEVTMP,NFRMAT,NDEVGRP,NDEVBIN

COMMON /DD/ INT,IOR,IT,IS,IC,ICC,IX,IY

C**** The following variables were added to handle the "buffering" of all

C     the "lineto" data i.e. calls to DDVC_ACUM.

C     After each "moveto", the x and y values for the "lineto"s

C     will be stored in IXTMP and IYTMP.  Then, a single write to NDEVGRP

c     is performed which writes all NUMPTS values.  NUMPTS is then reset

C     and the process starts over.

integer*2 numpts, ixtmp(3000), iytmp(3000)

character*12 lastcmd

C.....OUTRANGE handles cases when the moveto is off the plotting area.

C     This may be followed by a lineto which is on the plotting area, so

C     we want to be sure that the part of the line that falls in the

C     plotting area will show up on the plot.

logical outrange

data lastcmd/'            '/

C DEFINE NOMINAL PLOTTING AREA(ZERO SUFFIXES) AND AN

C OUTER CLIPPING BOUNDARY BEYOND WHICH NO VECTORS EXTEND.

DATA XOLD,YOLD/0.0,0.0/

C 90% FOR Y RANGE

C

C          COMPUTE SCALE FACTORS

1 IF (RESET.EQ.0.d0) GO TO 5

RESET=0.d0

IF(APLT.EQ.2.d0) THEN

MRNGE=723

MINX0=165

MINY0=140

ELSE

MRNGE=813

MINX0=165

MINY0=140

ENDIF

if(aplt.eq.4.d0) miny0=80

C

MAXX0=MINX0+MRNGE

MAXY0=MINY0+MRNGE

C

XSCALE=(MAXX0-MINX0)/(XMAX0-XMIN0)

YSCALE=(MAXY0-MINY0)/(YMAX0-YMIN0)

XMIN=XMIN0

YMIN=YMIN0

XMAX=XMAX0

YMAX=YMAX0

IF(APLT.EQ.2.d0) GO TO 5

C

XMIN=-ALPHA

XMAX=ALPHA

YMIN=RMIN

YMAX=RMAX

IF(APLT.NE.4.d0) GO TO 5

YSCALE=.85d0*YSCALE

MINY0=MINY0+60

C

C          START A NEW LINE

C          HORIZONTAL DISPLACEMENT

5     XS=X-XOLD

YS=Y-YOLD

S=1.d0

IF(NEW.EQ.0) GO TO 10

IF(X.GE.XMIN.AND.X.LE.XMAX.AND.Y.GE.YMIN.AND.Y.LE.YMAX) then

GO TO 48

else

outrange=.true.

GO TO 50

endif

C

10    IF (XS) 11,12,16

C          NEGATIVE

11 X1=XMAX

X2=XMIN

GO TO 20

C          ZERO

12 IF (YS) 13,50,14

13 S1=(YMAX-YOLD)/YS

S2=(YMIN-YOLD)/YS

GO TO 40

14 S1=(YMIN-YOLD)/YS

S2=(YMAX-YOLD)/YS

GO TO 40

C          POSITIVE

16 X1=XMIN

X2=XMAX

C

C          VERTICAL DISPLACEMENT

20 IF (YS) 21,22,26

C          NEGATIVE

21 Y1=YMAX

Y2=YMIN

GO TO 30

C          ZERO

22 S1=(X1-XOLD)/XS

S2=(X2-XOLD)/XS

GO TO 40

C          POSITIVE

26 Y1=YMIN

Y2=YMAX

C

30 S1=DMAX1((X1-XOLD)/XS,(Y1-YOLD)/YS)

S2=DMIN1((X2-XOLD)/XS,(Y2-YOLD)/YS)

C

C          PLOT LINE -- CHECKING FOR BORDER CROSSINGS

40    IF (S2.LT.0.d0.OR.S1.GT.1.d0) then

GO TO 50

endif

IF (S1.LT.0.d0) GO TO 42

C          PREVIOUS POINT OFF GRAPH

XP=XOLD+XS*S1

YP=YOLD+YS*S1

IF(APLT.EQ.2.d0.OR.APLT.EQ.4.d0) GO TO 41

T=XP

XP=YP*dSIN(T)

YP=YP*dCOS(T)

C

41    DDHIX=MINX0+(XP-XMIN0)*XSCALE+0.5d0

DDHIY=MINY0+(YP-YMIN0)*YSCALE+0.5d0

C     USE SPECIAL HI-REZ MODE

IX=REZFLG

if (outrange .and. lastcmd.eq.'            ') then

WRITE (lastcmd,333) 1,idint(1.d1*ddhix),

&                           idint(1.d1*ddhiy)

endif

GO TO 48

C

42    IF (S2.GT.1.d0) GO TO 48

C          CURRENT POINT OFF GRAPH

S=S2

C          CURRENT POINT ON GRAPH

48    XP=XOLD+XS*S

YP=YOLD+YS*S

IF(APLT.EQ.2.d0.OR.APLT.EQ.4.d0) GO TO 49

T=XP

XP=YP*dSIN(T)

YP=YP*dCOS(T)

49    DDHIX=MINX0+(XP-XMIN0)*XSCALE+0.5d0

DDHIY=MINY0+(YP-YMIN0)*YSCALE+0.5d0

C     USE SPECIAL HI-REZ MODE

IX=REZFLG

ccccc    IF(NEW.EQ.0) CALL DDVC

C The basic idea here is that when NEW = 0,

C the program accumulates "lineto" (i.e. DDVC) commands in IXTMP,IYTMP.

C Then, when NEW is not equal to 0, we are beginning a new line.  This

C requires a "moveto" (i.e."1 ix iy") command.  It is necessary, when a new

C "moveto" occurs that all "lineto"s that have been accumulated be

C "flushed".

C Occasionally, the plotting routines write some redundant "moveto"s, i.e.

C a "moveto" followed immediately by another "moveto" (sometimes to the

C same location!) with NO intervening "lineto"s.  This results in NUMPTS=

0.

C We ignore these movetos and do not flush.

C

C.....Handle very large plots. The IXTMP, IYTMP arrays are only dimensioned

C     to 3000.    This reduces the size of the executable file, decreases

C     the amount of memory used and increases execution speed.

C     Therefore, when 3000 is reached and the line is not yet finished,

C     write the 3000 pairs that have been accumulated.

if (new.eq.0 ) then

call DDVC_ACUM (numpts,ixtmp,iytmp)

if (numpts .eq. 3000) then

WRITE (ndevgrp,444) lastcmd

WRITE (ndevgrp,99) 3000

do ii = 1, 3000-7,8               !  3000/8 pairs per row =

!  375 rows

WRITE (ndevgrp,111) (ixtmp(j),iytmp(j),j=ii,ii+7)

enddo

C...............Save the last point as a moveto command in LASTCMD so we

can

C               pick up where we left off.

WRITE (lastcmd,333) 1,ixtmp(3000),iytmp(3000)

numpts = 0

endif

endif

C

C.....The call to DDBP only sets values of DDHIX,DDHIY, IX, IY.

C     It does not write to NDEVGRP.

IF(NEW.NE.0) then

call DDBP

C.........If we have accumulated any points, flush IXTMP, IYTMP.

if (numpts .gt.0) then

WRITE (ndevgrp,444) lastcmd    ! the moveto command

WRITE (ndevgrp,99) numpts

nremain = MOD(numpts,8)

if (numpts .ge. 8) then

do ii = 1, numpts-7, 8

WRITE (ndevgrp,111)(ixtmp(j),iytmp(j),j=ii,ii+7)

enddo

endif

C............if NUMPTS was not evenly divisible by 8, then the

C............remaining points are written to the file one per line.

if (nremain.gt.0) then

WRITE (ndevgrp,222)

&                (ixtmp(k),iytmp(k),k=numpts-nremain+1,numpts)

endif

endif

C.........Store the moveto command whenever NUMPTS = 0, or

C         whenever NEW is not 0 (i.e. a moveto occurs).  Either way, reset

C         NUMPTS.

C.........We write the moveto to a character string which is later written

C         to NDEVGRP.

WRITE (lastcmd,333) 1,idint(1.d1*ddhix), idint(1.d1*ddhiy)

numpts = 0   ! reset for DDVC_ACUM

endif  !    new .ne. 0

99    FORMAT (I5)

111   FORMAT (8(2I5))

222   FORMAT (2I5)

333   FORMAT (I2,2I5)

444   format (a)

555   format (1x,a)

C          EXIT ROUTINE

50 XOLD=X

YOLD=Y

C.....Handle a moveto that is off the plotting area.

if (outrange .and.new.ne.0)then

if (numpts.gt.0) then

WRITE (ndevgrp,444) lastcmd

WRITE (ndevgrp,99) numpts

nremain = MOD(numpts,8)

if (numpts .ge. 8) then

do ii = 1, numpts-7, 8

WRITE (ndevgrp,111)(ixtmp(j),iytmp(j),j=ii,ii+7)

enddo

endif

C..........if NUMPTS was not evenly divisible by 8, then the

C..........remaining points are written to the file one per line.

if (nremain.gt.0) then

WRITE (ndevgrp,222)

&                    (ixtmp(k),iytmp(k),k=numpts-nremain+1,numpts)

endif

numpts = 0

lastcmd = '            '

endif

endif

RETURN

C

C          TERMINATE THE CURRENT PLOT

C**********************************************************************

entry pltend

C**********************************************************************

C      ENTRY PLTEND(X,Y,NEW)

CALL DDFR

C

RETURN

END

Appendix B.  Contents of the Distribution Disk

PSGRAPH1.FOR

PSGRAPH2.FOR  - These two source code files comprise the entire source code

for the PSGRAPH program.

PSGRAPH.EXE   - The executable file

RUNPS.BAT     - A DOS batch file for running PSGRAPH with either

interactive input or redirected input.

TXTOUT        - Input data file for sample case 1, Profile Plots.

TXTOUT2       - Input data file for sample case 2, Ray Plots.

TXTOUT3       - Input data file for sample case 3, Contour Plots.

TXTOUT4       - Input data file for sample case 4, Eigen Plots.

TXTOUT5       - Input data file for sample case 5, Eigenray Plot.

PS.SAM        - Output PostScript file for sample case 1.

PS2.SAM       - Output PostScript file for sample case 2.

REDIRECT.SAM  - Example file for redirection of standard input

PSGRAPH.DOC   - The document you are reading.

CLRS.EXE

CLRS.FOR      - Source code and executable file for the program

that returns the video screen to text mode.

SHOWMODE.EXE

SHOWMODE.FOR  - Source code and executable file for a program

that displays the current video mode.

Appendix C1.  Input Data File Listing for Sample Case 1

MIKE=JONES=RL3=X6464

0T6  MUNK'S 1974 UNIFORM SOUND CHANNEL WITH VVORTEX CURRENT MODEL 3

29 1.                  1=PROCESS PROFILE RUNSET, 0=SKIP RUNSET

83 0.           AN KM  LEFT LATITUDE OF PLOT, KM

84 0.           AN KM  LEFT LONGITUDE OF PLOT, KM

88-5.00000000000       HEIGHT ABOVE SEA LEVEL OF BOTTOM OF GRAPH, KM

89 0.                  HEIGHT ABOVE SEA LEVEL OF THE TOP OF THE GRAPH

90 1.00000000000       SELECT SOUND SPEED

92 1.49000000000       HORIZONTAL AXIS MINIMUM

93 1.54000000000       HORIZONTAL AXIS MAXIMUM

94 .010000000000LN KM  HORIZONTAL AXIS TICK MARK INTERVAL

96 1.00000000000       DISTANCE BETWEEN VERTICAL TICK MARKS, KM

100        9.           VVORTX3 MODEL CHECK NUMBER

-1                     DATA SUBSET FOR BACKGROUND CURRENT MODEL

A  VORTEX AT LONGITUDE 150 KM E, UMAX= 1.02 M/S, R= 50 KM

0        RETURN TO W-ARRAY DATA SET

102        3.            VVORTX3 BACKGROUND CURRENT DATA SET ID

103      1.02    LN  M   MAXIMUM TANGENTIAL CURRENT, M/S

104       50.            RADIUS OF VORTEX CORE, KM

105        0.    AN KM   LATITUDE OF VORTEX CENTER, KM

106      150.    AN KM   LONGITUDE OF VORTEX CENTER, KM

107        1.            VERTICAL HALF-WIDTH OF VORTEX, KM

108       -1.            HEIGHT OF VORTEX CENTER ABOVE MSL, KM

125        0.           NPCURR MODEL CHECK NUMBER

-2                     DATA SUBSET FOR PERTURBATION CURRENT MODEL

A  NO CURRENT PERTURBATION

0        RETURN TO W-ARRAY DATA SET

150 5.00000000000       CSMUNK1 SOUND SPEED MODEL

152 2.00000000000       INPUT DATA SET ID NUMBER

153 1.00000000000       REFERENCE SOUND SPEED

154 0.           AN KM  PH1 LONGITUDE 1

155 1.49200000000       CA1 SOUND SPEED ON AXIS

156-1.30000000000       ZA1 DEPTH OF AXIS

157 1.30000000000       H1 SCALE DEPTH

158 .007400000000       EP1 FRACTIONAL INCREASE OF C WITH DEPTH

159 1000.00000000AN KM  PH2 LONGITUDE 2

160 1.49200000000       CA2 SOUND SPEED ON AXIS

161-1.30000000000       ZA2 DEPTH OF AXIS

162 1.30000000000       H2 SCALE DEPTH

163 .007400000000       EP2 FRACTIONAL INCREASEE OF C WITH DEPTH

175 0.0                 NPSPEED - DO NOTHING SPEED PERTURBATION MODEL

200 1.0                 TLINEAR MODEL CHECK NUMBER

201 1.                  DATA FORMAT CODE

202 1.                  DATA SET ID NUMBER

203 293.                BOTTOM TEMPERATURE, DEGREES KELVIN

204 2.3                 TEMPERATURE GRADIENT, DEGREES KELVIN/KM

225 0.                  NPTEMP, DO-NOTHING TEMPERATURE PERTURBATION

275 1.00000000000       RHORIZ RECEIVER MODEL CHECK NUMBER

300        4.           GLORENZ BOTTOM MODEL CHECK NUMBER

302        3.            GLORENZ BOTTOM MODEL DATA SET ID

303        .5            HEIGHT OF RIDGE, KM ABOVE BASE

304       10.    AN KM   N. LATITUDE OF RIDGE CENTER, KM

305        2.    AN KM   HALF-WIDTH OF THE RIDGE, KM

306       -3.            HEIGHT ABOVE MSL OF BASE OF RIDGE, KM

325        0.            DO-NOTHING BOTTOM PERTURBATION

0

0T6    SAMPLE SOUND SPEED DATA POINTS

90-42.0000000000       NEGATIVE VALUE = SUPERIMPOSE DATA POINTS

-42                     ENTERING DATA SUBSET

2      999.0

LN KM        LN KM

-.568240E-01  1.53400

-.710503E-01  1.53282

-.853171E-01  1.53165

-.995969E-01  1.53048

-.113850      1.52931

-.128006      1.52814

-.141939      1.52697

-.155372      1.52581

-.167350      1.52464

-.177608      1.52348

-.195094      1.52232

-.212698      1.52117

-.230376      1.52001

-.248050      1.51886

-.265561      1.51770

-.282521      1.51655

-.297255      1.51540

-.313064      1.51426

-.335598      1.51311

-.358481      1.51197

-.381655      1.51082

-.404971      1.50968

-.428009      1.50854

-.448662      1.50741

-.471800      1.50627

-.503609      1.50514

-.536853      1.50401

-.571848      1.50288

-.609165      1.50175

-.650365      1.50062

-.698865      1.49950

-.733760      1.49837

-.770014      1.49725

-.806911      1.49613

-.841016      1.49501

-.843638      1.49390

-.941190      1.49278

-4.32248      1.53400

-4.26850      1.53282

-4.21328      1.53165

-4.15665      1.53048

-4.09840      1.52931

-4.03821      1.52814

-3.97557      1.52697

-3.90943      1.52581

-3.83637      1.52464

-3.75539      1.52348

-3.69719      1.52232

-3.63710      1.52117

-3.57480      1.52001

-3.50979      1.51886

-3.44119      1.51770

-3.36717      1.51655

-3.28030      1.51540

-3.19451      1.51426

-3.13375      1.51311

-3.07037      1.51197

-3.00377      1.51082

-2.93298      1.50968

-2.85600      1.50854

-2.76490      1.50742

-2.67538      1.50627

-2.45335      1.50288

-2.36103      1.50175

-2.24327      1.50062

-2.06858      1.49950

-2.03932      1.49837

-2.01206      1.49725

-1.99054      1.49613

-1.98684      1.49501

-2.08138      1.49390

-1.97047      1.49278

999.000

0                     RETURNING TO W ARRAY DATA SET

0

0T6    SAMPLE SOUND SPEED DATA POINTS

90-41.0000000000       NEGATIVE VALUE = SUPERIMPOSE ON PREVIOUS PLOT

-41                     NEGATIVE VALUE = TABULAR DATA SET FOLLOWS

2      999.0

LN KM        LN KM

-2.66103      1.51175

-2.54327      1.51062

-2.40858      1.50950

-2.40932      1.50837

-2.40206      1.50725

-2.38054      1.50613

-2.28684      1.50501

-2.30138      1.50390

-2.27047      1.50278

999.000

0                     RETURNING TO W ARRAY DATA SET

0

0T6  MUNK'S 1974 UNIFORM SOUND CHANNEL WITH VVORTEX CURRENT MODEL 3

29 1.0000              1 = PROFILE, 0 = SKIP THIS RUNSET

83 5.           AN KM  Latitude of the profile

84 145.         AN KM  Longitude of the profile

88-5.00000000000       Height above sea level of bottom of graph km

89 0.                  HEIGHT ABOVE SEA LEVEL OF TOP OF GRAPH

90 2.00000000000       CURRENT SPEED

92 0.00000000000       Minimum speed km/s, m/s

93 0.00000000000       Maximum speed km/s, m/s

94 0.00000000000       Horizontal tick mark interval km/s, m/s

96 1.00000000000       Vertical tick mark interval km

0

0T6  MUNK'S 1974 UNIFORM SOUND CHANNEL WITH VVORTEX CURRENT MODEL 3

29 1.0000              1 = PROFILE, 0 = SKIP THIS RUNSET

83 5.           AN KM  Latitude of the profile   (rad, deg, km) north

84 145.         AN KM  Longitude of the profile   (rad, deg, km) east

88-5.00000000000       Height above sea level of bottom of graph   km

89    0.               Height above sea level of top of graph  km

90 4.00000000000       SOUTHWARD component of current velocity

92 0.00000000000       Minimum speed km/s, m/s

93 0.00000000000       Maximum speed km/s, m/s

94 0.00000000000       Horizontal tick mark interval km/s, m/s

96 0.00000000000       Vertical tick mark interval km

0

0T6  MUNK'S 1974 UNIFORM SOUND CHANNEL WITH VVORTEX CURRENT MODEL 3

29 1.0000              1 = PROFILE, 0 = SKIP THIS RUNSET

83 5.           AN KM  Latitude of the profile  (rad, deg, km) north

84 145.         AN KM  Longitude of the profile  (rad, deg, km) east

88-5.00000000000       Height above sea level of bottom of graph  km

89 0.                  Height above sea level of top of graph  km

90 5.00000000000EASTWARD component of current velocity

92 0.00000000000       Minimum speed  km/s, m/s

93 0.00000000000       Maximum speed  km/s, m/s

94 0.00000000000       Horizontal tick mark interval km/s, m/s

96 1.00000000000Vertical tick mark interval km

0

0T6    TLINEAR TEMPERATURE PROFILE

29 1.0000              1 = PROFILE, 0 = SKIP THIS RUNSET

83 5.           AN KM  Latitude of the profile   (rad, deg, km) north

84 145.         AN KM  Longitude of the profile   (rad, deg, km) east

88 0.00000000000       Height above sea level of bottom of graph   km

89    3.               Height above sea level of top of graph  km

90 7.00000000000       TEMPERATURE PROFILE

92 290.000000000       Minimum temperature deg K

93 300.000000000       Maximum temperature deg K

94 0.00000000000       Horizontal tick mark interval deg K

96 0.00000000000       Vertical tick mark interval km

0

0T6    SAMPLE TEMPERATURE DATA POINTS

90-45.0000000000       NEGATIVE VALUE = SUPERIMPOSE ON PREVIOUS PLOT

-45                     NEGATIVE VALUE = TABULAR DATA SET FOLLOWS

2      999.0

1.06103      295.1175

1.04327      295.1062

1.00858      295.0950

1.10932      295.0837

1.20206      295.0725

1.38054      296.0613

1.28684      296.0501

1.40138      296.0390

1.47047      296.0278

999.000

0                     RETURNING TO W ARRAY DATA SET

0

0T6    GLORENZ TOPOGRAPHY PROFILE

29 1.0000              1 = PROFILE, 0 = SKIP THIS RUNSET

83 0.           AN KM  Left Latitude of plot   (rad, deg, km) north

84 0.           AN KM  Longitude of plot  (rad, deg, km) east

85 20.          AN KM  Right Latitude of plot  (rad, deg, km) north

90 6.00000000000       TOPOGRAPHY PROFILE

0

Appendix C2.  Input Data File Listing for Sample Case 2

GEORGES   RB3    X6437

N01-Sample Case 2, Plot 1:  PSGRAPH Documentation

1     6370.           EARTH RADIUS TO MSL, KM (6370.)

3        2.          TTRANSMITTER HEIGHT ABOVE MSL (T=ABOVE BOTM),KM

4        0.    AN KM  N. TRANSMITTER LATITUDE, KM

5        0.    AN KM  E. TRANSMITTER LONGITUDE, KM

7      400.    FQ HZ  INITIAL FREQUENCY, HZ

11       80.    AN DG  INITIAL AZIMUTH ANGLE, DEG

15        2.    AN DG  INITIAL ELEVATION ANGLE, DEG

16        4.    AN DG  FINAL   ELEVATION ANGLE, DEG

17        2.    AN DG  STEP IN ELEVATION ANGLE, DEG

19        0.           STOP RAYS THAT STRIKE BOTTOM (1=YES; 0=NO)

20       -1.           RECEIVER HEIGHT ABOVE MSL, KM

22       50.           MAXIMUM NUMBER OF HOPS (1.)

23     1000.           MAXIMUM NUMBER OF STEPS PER HOP (1000.)

26        5.           MAXIMUM RAY HEIGHT ABOVE MSL, KM

27       -5.           MINIMUM RAY HEIGHT ABOVE MSL, KM

28      210.           MAXIMUM RANGE AT MSL, KM

29        0.           DO:RAYTRC

33       20.           MAXIMUM ABSORPTION, DB (999.999)

42       1.0E-03       MAXIMUM SINGLE-STEP INTEGRATION ERROR (1.0E-4)

44        .1           INITIAL INTEGRATION STEP SIZE, KM (1.0)

57        2.           PHASE PATH  (0=NO;1=INTEGRATE;2=INTEGR/PRINT)

58        2.           ABSORPTION  (0=NO;1=INTEGRATE;2=INTEGR/PRINT)

60        2.           PATH LENGTH (0=NO;1=INTEGRATE;2=INTEGR/PRINT)

71       50.           NUMBER OF INTEGRATION STEPS PER PRINT [1.E9]

72        1.           OUTPUT RAYSETS (1=YES; 0=NO)

73        0.           DIAGNOSTIC PRINTOUT (1=YES; 0=NO)

74        0.           PRINT EVERY W(71) RAY STEPS (0=YES; 1=N0)

76        1.           BINARY RAY OUTPUT (1=YES; 0=NO)

77       76.           LINES PER PAGE OF PRINTOUT= 76. FOR HPLJ (57.)

81        4.           RAYPLOT PROJECTION PLANE (2 = HORIZONTAL)

82       40.           PLOT-ORDINATE EXPANSION FACTOR [1.]

83        0.    AN KM  N. LATITUDE OF LEFT PLOT EDGE, KM

84        0.    AN KM  E. LONGITUDE OF LEFT PLOT EDGE, KM

85  35.265396   AN KM  N. LATITUDE OF RIGHT PLOT EDGE, KM

86      200.    AN KM  E. LONGITUDE OF RIGHT PLOT EDGE, KM

87       50.    AN KM  DISTANCE BETWEEN RANGE TICKS, KM

88       -3.           HEIGHT ABOVE MSL OF BOTTOM OF GRAPH, KM

89        0.           HEIGHT ABOVE MSL OF TOP OF GRAPH, KM

96        1.           DISTANCE BETWEEN DEPTH TICKS, KM

100        9.           VVORTX3 MODEL CHECK NUMBER

102        3.           VVORTX3 BACKGROUND CURRENT DATA SET ID

103      1.02    LN  M  MAXIMUM TANGENTIAL CURRENT, M/S

104       50.           RADIUS OF VORTEX CORE, KM

105        0.    AN KM  LATITUDE OF VORTEX CENTER, KM

106      150.    AN KM  LONGITUDE OF VORTEX CENTER, KM

107        1.           VERTICAL HALF-WIDTH OF VORTEX, KM

108       -1.           HEIGHT OF VORTEX CENTER ABOVE MSL, KM

125        0.           NPCURR MODEL CHECK NUMBER

150        7.           CTANH SOUND SPEED MODEL CHECK NUMBER

152        1.           CTANH BACKGROUND SOUND SPEED DATA SET ID

175        2.           CBLOB2 SOUND SPEED PERTURB MODEL CHECK NUM

177        7.           CBLOB2 PERTURBATION SOUND SPEED DATA SET ID

178       .02           MAXIMUM FRACTIONAL INCREASE IN C SQUARED

179       -1.           HEIGHT OF MAX EFFECT ABOVE MSL, KM

180        0.    AN KM  LATITUDE OF MAX EFFECT, KM

181      150.    AN KM  LONGITUDE OF MAX EFFECT, KM

182        1.           VERTICAL HALF-WIDTH, KM

183       50.    AN KM  N-S  HALF-WIDTH, KM

184       50.    AN KM  E-W  HALF-WIDTH, KM

275        1.           RHORIZ RECEIVER MODEL CHECK NUMBER

300        4.           GLORENZ BOTTOM MODEL CHECK NUMBER

302        3.            GLORENZ BOTTOM MODEL DATA SET ID

303        .5            HEIGHT OF RIDGE, KM ABOVE BASE

304       10.    AN KM   N. LATITUDE OF RIDGE CENTER, KM

305        2.    AN KM   HALF-WIDTH OF THE RIDGE, KM

306       -3.            HEIGHT ABOVE MSL OF BASE OF RIDGE, KM

325        0.           NPBOTM MODEL CHECK NUMBER

350        1.           SHORIZ MODEL CHECK NUMBER

352        1.            SHORIZ OCEAN SURFACE DATA SET ID

353        0.            HEIGHT OF OCEAN SURFACE ABOVE MSL, KM

375        0.           NPSURF MODEL CHECK NUMBER

500        1.           SLLOSS ABSORPTION MODEL CHECK NUMBER

502        1.            SLLOSS ABSORPTION DATA SET ID

503     0.006    AM DB   A COEFFICIENT, DB

504    0.2635    AM DB   B COEFFICIENT, DB

505     1000.    FQ HZ   OMEGA1, HZ

506     1700.    FQ HZ   OMEGA2, HZ

525        0.           NPABSRP MODEL CHECK NUMBER

-1                     DATA SUBSET FOR BACKGROUND CURRENT MODEL

A  VORTEX AT LONGITUDE 150 KM E, UMAX= 1.02 M/S, R= 50 KM

0        RETURN TO W-ARRAY DATA SET

-2                     DATA SUBSET FOR PERTURBATION CURRENT MODEL

A  NO CURRENT PERTURBATION

0        RETURN TO W-ARRAY DATA SET

-3                     DATA SUBSET FOR BACKGROUND SOUND-SPEED MODEL

A  EL NINO BACKGROUND SOUND-SPEED PROFILE

3     999.0

LN  M       LN  M       LN  M

0.          1532.       0.

-20.        1531.5      -7.

-50.        1509.       -20.

-250.       1503.       -40.

-450.       1485.       -300.

-1500.      1485.       -400.

-3000.      1508.       0.

999.0

0       RETURN TO W-ARRAY DATA SET

-4                     DATA SUBSET FOR SOUND-SPEED PERTURBATION MODEL

A  2% INCREASE IN C-SQUARED AT 150 KM LON., 1 KM DEPTH, 50 KM WIDE

0       RETURN TO W-ARRAY DATA SET

-8                     DATA SUBSET FOR RECEIVER-SURFACE MODEL

A  RECEIVER SURFACE = SPHERE 1 KM BELOW MSL

0       RETURN TO W-ARRAY DATA SET

-9                     DATA SUBSET FOR BACKGROUND BOTTOM MODEL

A  RIDGE .5 KM HIGH, 2 KM WIDE AT 10 KM N LATITUDE; BASE= -3 KM

0       RETURN TO W-ARRAY DATA SET

-10                     DATA SUBSET FOR BOTTOM PERTURBATION MODEL

A  NO BOTTOM PERTURBATION

0       RETURN TO W-ARRAY DATA SET

-11                     DATA SUBSET FOR OCEAN SURFACE MODEL

A  OCEAN SURFACE = SPHERE AT MSL

0       RETURN TO W-ARRAY DATA SET

-12                     DATA SUBSET FOR OCEAN SURFACE PERTURBA MODEL

A  NO OCEAN SURFACE PERTURBATION

0       RETURN TO W-ARRAY DATA SET

-17                     DATA SUBSET FOR OCEAN ABSORPTION MODEL

A  SKRETTING-LEROY ABSORPTION FORMULA

0       RETURN TO W-ARRAY DATA SET

-18                     DATA SUBSET FOR PERTURBATION ABSORPTION MODEL

A  NO ABSORPTION PERTURBATION

0       RETURN TO W-ARRAY DATA SET

0                     **********  END OF RUN SET NUMBER 1  *********

N01-Sample Case 2, Plot 2: PSGRAPH Documentation

29      0.             DO: RAYTRC

81      2.             HORIZONTAL PLOT PROJECTION

82      80.            PLOT-ORDINATE EXPANSION FACTOR [1.]

88     -1.             DEPTH OF HORIZONTAL PLANE, KM

0                      ***********  END OF RUN SET NUMBER 2  ********

N01-Sample Case 2, Plot 3: PSGRAPH Documentation

29       0.            DO: RAYTRC

81        4.           RAYPLOT PROJECT. PLANE (4 = VERT. RECTANGULAR)

82      180.           PLOT-ORDINATE EXPANSION FACTOR [1.]

83    1.00000   AN KM  N. LATITUDE OF LEFT PLOT EDGE, KM

84  -.1763269   AN KM  E. LONGITUDE OF LEFT PLOT EDGE, KM

85   -1.00000   AN KM  N. LATITUDE OF RIGHT PLOT EDGE, KM

86   .1763269   AN KM  E. LONGITUDE OF RIGHT PLOT EDGE, KM

87       1.     AN KM  DISTANCE BETWEEN RANGE TICKS, KM

88       -3.           HEIGHT ABOVE MSL OF BOTTOM OF GRAPH, KM

0                      ***********  END OF RUN SET NUMBER 3  ********

N01-Sample Case 2, Plot 4: PSGRAPH Documentation

29        0.           DO: RAYTRC

81        3.           RAYPLOT PROJECT. PLANE (4 = VERT. RECTANGULAR)

82      200.           PLOT-ORDINATE EXPANSION FACTOR [1.]

83        0.    AN KM  N. LATITUDE OF LEFT PLOT EDGE, KM

84        0.    AN KM  E. LONGITUDE OF LEFT PLOT EDGE, KM

85 141.0615846  AN KM  N. LATITUDE OF RIGHT PLOT EDGE, KM

86      800.    AN KM  E. LONGITUDE OF RIGHT PLOT EDGE, KM

87      200.    AN KM  DISTANCE BETWEEN RANGE TICKS, KM

88       -3.            HEIGHT ABOVE MSL OF BOTTOM OF GRAPH, KM

89        0.            HEIGHT ABOVE MSL OF TOP OF GRAPH, KM

96        1.            DISTANCE BETWEEN DEPTH TICKS, KM

28      800.           MAXIMUM RANGE AT MSL, KM

0                      ***********  END OF RUN SET NUMBER 4  ********

Appendix C3.  Input Data File Listing for Sample Case 3

MIKE JONES RL3 X6464

N01-1     Sample Case for CONPLT Documentation

1 6370.00000000       EARTH RADIUS TO MSL, KM (6370.)

3 2.00000000000      TTRANSMITTER HEIGHT ABOVE MSL (T=ABOVE BOTM),KM

4 0.           AN KM  N. TRANSMITTER LATITUDE, KM

5 0.           AN KM  E. TRANSMITTER LONGITUDE, KM

7 400.000000000FQ HZ  INITIAL FREQUENCY, HZ

11 80.0000000000AN DG  INITIAL AZIMUTH ANGLE, DEG

15 2.00000000000AN DG  INITIAL ELEVATION ANGLE, DEG

16 16.0000000000AN DG  FINAL   ELEVATION ANGLE, DEG

17 2.00000000000AN DG  STEP IN ELEVATION ANGLE, DEG

19 0.                  STOP RAYS THAT STRIKE BOTTOM (1=YES; 0=NO)

20-1.00000000000       RECEIVER HEIGHT ABOVE MSL, KM

22 50.0000000000       MAXIMUM NUMBER OF HOPS (1.)

23 1000.00000000       MAXIMUM NUMBER OF STEPS PER HOP (1000.)

26 5.00000000000       MAXIMUM RAY HEIGHT ABOVE MSL, KM

27-5.00000000000       MINIMUM RAY HEIGHT ABOVE MSL, KM

28 210.000000000       MAXIMUM RANGE AT MSL, KM

29 1.                  1=PROCESS RUNSET, 0=SKIP RUNSET

33 20.0000000000       MAXIMUM ABSORPTION, DB (999.999)

42  .1000000E-05       MAXIMUM SINGLE-STEP INTEGRATION ERROR (1.0E-4)

44 .100000000000       INITIAL INTEGRATION STEP SIZE, KM (1.0)

57 2.00000000000       PHASE PATH  (0=NO;1=INTEGRATE; 2=INTEGR/PRINT)

58 2.00000000000       ABSORPTION  (0=NO;1=INTEGRATE; 2=INTEGR/PRINT)

60 2.00000000000       PATH LENGTH (0=NO;1=INTEGRATE; 2=INTEGR/PRINT)

71 50.0000000000       NUMBER OF INTEGRATION STEPS PER PRINT [1.E9]

72 1.00000000000       OUTPUT RAYSETS (1=YES; 0=NO)

73 0.                  DIAGNOSTIC PRINTOUT (1=YES; 0=NO)

74 0.                  PRINT EVERY W(71) RAY STEPS (0=YES; 1=N0)

76 0.                  BINARY RAY OUTPUT (1=YES; 0=NO)

77 76.0000000000       LINES PER PAGE OF PRINTOUT= 76. FOR HPLJ (57.)

81 4.00000000000       RAYPLOT PROJECT. PLANE (4 = VERT. RECTANGULAR)

82 40.00000000000      PLOT-ORDINATE EXPANSION FACTOR [1.]

83 0.           AN KM  N. LATITUDE OF LEFT PLOT EDGE, KM

84 0.           AN KM  E. LONGITUDE OF LEFT PLOT EDGE, KM

85  35.265396   AN KM  N. LATITUDE OF RIGHT PLOT EDGE, KM

86 200.000000000AN KM  E. LONGITUDE OF RIGHT PLOT EDGE, KM

87 50.0000000000AN KM  DISTANCE BETWEEN RANGE TICKS, KM

88-3.00000000000       HEIGHT ABOVE MSL OF BOTTOM OF GRAPH, KM

89 0.00                HEIGHT ABOVE MSL OF TOP OF GRAPH, KM

90 1.00000000000       SOUND SPEED CONTOURS

92 1486.00000000LN  M  MINIMUM CONTOUR LEVEL

93 1540.00000000LN  M  MAXIMUM CONTOUR LEVEL

94 4.00000000000LN  M  CONTOUR INTERVAL

96 1.00000000000       DISTANCE BETWEEN DEPTH TICKS, KM

100 9.00000000000       VVORTX3 MODEL CHECK NUMBER

102 3.00000000000       VVORTX3 BACKGROUND CURRENT DATA SET ID

103 1.02000000000LN  M  MAXIMUM TANGENTIAL CURRENT, M/S

104 50.0000000000       RADIUS OF VORTEX CORE, KM

105 0.           AN KM  LATITUDE OF VORTEX CENTER, KM

106 150.000000000AN KM  LONGITUDE OF VORTEX CENTER, KM

107 1.00000000000       VERTICAL HALF-WIDTH OF VORTEX, KM

108-1.00000000000       HEIGHT OF VORTEX CENTER ABOVE MSL, KM

125 0.                  NPCURR MODEL CHECK NUMBER

150 7.00000000000       CTANH SOUND SPEED MODEL CHECK NUMBER

152 1.00000000000       CTANH BACKGROUND SOUND SPEED DATA SET ID

175 2.00000000000       CBLOB2 SOUND SPEED PERTURB MODEL CHECK NUM

177 7.00000000000       CBLOB2 PERTURBATION SOUND SPEED DATA SET ID

178 .020000000000       MAXIMUM FRACTIONAL INCREASE IN C SQUARED

179-1.00000000000       HEIGHT OF MAX EFFECT ABOVE MSL, KM

180 0.           AN KM  LATITUDE OF MAX EFFECT, KM

181 150.000000000AN KM  LONGITUDE OF MAX EFFECT, KM

182 1.00000000000       VERTICAL HALF-WIDTH, KM

183 50.0000000000AN KM  N-S  HALF-WIDTH, KM

184 50.0000000000AN KM  E-W  HALF-WIDTH, KM

275 1.00000000000       RHORIZ RECEIVER MODEL CHECK NUMBER

300 4.0                 GLORENZ BOTTOM MODEL CHECK NUMBER

301 0.                  DATA INPUT FORMAT CODE NUMBER

302 0.                  DATA SET IDENTIFICATION NUMBER

303 1.00                HEIGHT OF RIDGE (KM)

304 10.0         AN KM  LATITUDE OF THE RIDGE CENTER

305 2.0          AN KM  WIDTH OF RIDGE (KM)

306-3.0                 BASE OF RIDGE  (KM MSL)

325 0.0                 NPBOTM MODEL CHECK NUMBER

350 1.00000000000       SHORIZ MODEL CHECK NUMBER

352 1.00000000000       SHORIZ OCEAN SURFACE DATA SET ID

353 0.                  HEIGHT OF OCEAN SURFACE ABOVE MSL, KM

375 0.                  NPSURF MODEL CHECK NUMBER

500 1.00000000000       SLLOSS ABSORPTION MODEL CHECK NUMBER

502 1.00000000000       SLLOSS ABSORPTION DATA SET ID

503 .006000000000AM DB  A COEFFICIENT, DB

504 .263500000000AM DB  B COEFFICIENT, DB

505 1000.00000000FQ HZ  OMEGA1, HZ

506 1700.00000000FQ HZ  OMEGA2, HZ

525 0.                  NPABSRP MODEL CHECK NUMBER

-1                     DATA SUBSET FOR BACKGROUND CURRENT MODEL

A  VORTEX AT LONGITUDE 150 KM E, UMAX= 1.02 M/S, R= 50 KM

0        RETURN TO W-ARRAY DATA SET

-2                     DATA SUBSET FOR PERTURBATION CURRENT MODEL

A  NO CURRENT PERTURBATION

0        RETURN TO W-ARRAY DATA SET

-3                     DATA SUBSET FOR BACKGROUND SOUND-SPEED MODEL

A  EL NINO BACKGROUND SOUND-SPEED PROFILE

3      999.0

LN  M       LN  M       LN  M

0.            1532.00     0.

-20.0000      1531.50     -7.00000

-50.0000      1509.00     -20.0000

-250.000      1503.00     -40.0000

-450.000      1485.00     -300.000

-1500.00      1485.00     -400.000

-3000.00      1508.00     0.

999.000

0       RETURN TO W-ARRAY DATA SET

-4                     DATA SUBSET FOR SOUND-SPEED PERTURBATION MODEL

A  2% INCREASE IN C-SQUARED AT 150 KM LON., 1 KM DEPTH, 50 KM WIDE

0       RETURN TO W-ARRAY DATA SET

-8                     DATA SUBSET FOR RECEIVER-SURFACE MODEL

A  RECEIVER SURFACE = SPHERE 1 KM BELOW MSL

0       RETURN TO W-ARRAY DATA SET

-9                     DATA SUBSET FOR BACKGROUND BOTTOM MODEL

A  RIDGE .5 KM HIGH, 2 KM WIDE AT 10 KM N LATITUDE; BASE= -3 KM

0       RETURN TO W-ARRAY DATA SET

-10                     DATA SUBSET FOR BOTTOM PERTURBATION MODEL

A  NO BOTTOM PERTURBATION

0       RETURN TO W-ARRAY DATA SET

-11                     DATA SUBSET FOR OCEAN SURFACE MODEL

A  OCEAN SURFACE = SPHERE AT MSL

0       RETURN TO W-ARRAY DATA SET

-12                     DATA SUBSET FOR OCEAN SURFACE PERTURB MODEL

A  NO OCEAN SURFACE PERTURBATION

0       RETURN TO W-ARRAY DATA SET

-17                     DATA SUBSET FOR OCEAN ABSORPTION MODEL

A  SKRETTING-LEROY ABSORPTION FORMULA

0       RETURN TO W-ARRAY DATA SET

-18                     DATA SUBSET FOR PERTURBATION ABSORPTION MODEL

A  NO ABSORPTION PERTURBATION

0       RETURN TO W-ARRAY DATA SET

0                     **********  END OF RUN SET NUMBER 1  *********

N01-2     Sample Case for CONPLT Documentation

29 1.                  1=PROCESS RUNSET, 0=SKIP RUNSET

81 4.00000000000       RAYPLOT PROJECTION PLANE (2 = HORIZONTAL)

82 40.0000000000        PLOT-ORDINATE EXPANSION FACTOR [1.]

85  35.265396   AN KM   N. LATITUDE OF RIGHT PLOT EDGE, KM

86 200.000000000AN KM   E. LONGITUDE OF RIGHT PLOT EDGE, KM

87 50.0000000000AN KM   DISTANCE BETWEEN RANGE TICKS, KM

88-3.00000000000        HEIGHT ABOVE MSL OF BOTTOM OF GRAPH, KM

89 0.0                  HEIGHT ABOVE MSL OF TOP OF GRAPH, KM

90 2.00000000000        CURRENT SPEED CONTOURS

92 0.10000000000LN  M   MINIMUM CONTOUR LEVEL

93 1.00000000000LN  M   MAXIMUM CONTOUR LEVEL

94 0.10000000000LN  M   CONTOUR INTERVAL

96 1.00000000000        DISTANCE BETWEEN DEPTH TICKS, KM

0                     **********  END OF RUN SET NUMBER 2  *********

N01-3     Sample Case for CONPLT Documentation

29 1.                  1=PROCESS RUNSET, 0=SKIP RUNSET

81 3.00000000000       RAYPLOT PROJECTION PLANE (2 = HORIZONTAL)

82 200.00000000        PLOT-ORDINATE EXPANSION FACTOR [1.]

85 141.0615846  AN KM  N. LATITUDE OF RIGHT PLOT EDGE, KM

86 800.000000000AN KM   E. LONGITUDE OF RIGHT PLOT EDGE, KM

87 100.000000000AN KM   DISTANCE BETWEEN RANGE TICKS, KM

88-3.00000000000        HEIGHT ABOVE MSL OF BOTTOM OF GRAPH, KM

90 1.0                 SOUND SPEED CONTOURS

92 1486.00000000LN  M  MINIMUM CONTOUR LEVEL

93 1540.00000000LN  M  MAXIMUM CONTOUR LEVEL

94 6.00000000000LN  M  CONTOUR INTERVAL

96 1.00000000000        DISTANCE BETWEEN DEPTH TICKS, KM

0                     **********  END OF RUN SET NUMBER 3  *********

N01-4     Sample Case for CONPLT Documentation

29 1.                  1=PROCESS RUNSET, 0=SKIP RUNSET

81 2.00000000000       RAYPLOT PROJECTION PLANE (2 = HORIZONTAL)

82 1.00000000000       PLOT-ORDINATE EXPANSION FACTOR [1.]

85  35.265396   AN KM  N. LATITUDE OF RIGHT PLOT EDGE, KM

86 200.000000000AN KM  E. LONGITUDE OF RIGHT PLOT EDGE, KM

87 50.0000000000AN KM  DISTANCE BETWEEN RANGE TICKS, KM

88-1.00000000000       HEIGHT OF HORIZONTAL PLOT SECTION ABOVE MSL,KM

90 2.0                 CURRENT SPEED CONTOURS

92 0.10000000000LN  M  MINIMUM CONTOUR LEVEL

93 1.00000000000LN  M  MAXIMUM CONTOUR LEVEL

94 0.10000000000LN  M  CONTOUR INTERVAL

96 100.000000000       DISTANCE BETWEEN DEPTH TICKS, KM

0                     **********  END OF RUN SET NUMBER 4  *********

N01-5     Sample Case for CONPLT Documentation

29 1.                  1=PROCESS RUNSET, 0=SKIP RUNSET

81 4.00000000000       RAYPLOT PROJECTION PLANE (2 = HORIZONTAL)

82 40.0000000000       PLOT-ORDINATE EXPANSION FACTOR [1.]

85  35.265396   AN KM  N. LATITUDE OF RIGHT PLOT EDGE, KM

86 200.000000000AN KM  E. LONGITUDE OF RIGHT PLOT EDGE, KM

87 50.0000000000AN KM  DISTANCE BETWEEN RANGE TICKS, KM

88-3.00000000000       HEIGHT ABOVE MSL OF BOTTOM OF GRAPH, KM

89 0.0                 HEIGHT ABOVE MSL OF TOP OF GRAPH, KM

90 4.00000000000       CURRENT SPEED CONTOURS

92-1.00000000000LN  M  MINIMUM CONTOUR LEVEL

93 1.00000000000LN  M  MAXIMUM CONTOUR LEVEL

94 0.10000000000LN  M  CONTOUR INTERVAL

96 1.00000000000       DISTANCE BETWEEN DEPTH TICKS, KM

0                     **********  END OF RUN SET NUMBER 5  *********

N01-6     Sample Case for CONPLT Documentation

29 1.                  1=PROCESS RUNSET, 0=SKIP RUNSET

81 2.00000000000       RAYPLOT PROJECTION PLANE (2 = HORIZONTAL)

82 5.00000000000       PLOT-ORDINATE EXPANSION FACTOR [1.]

85  35.265396   AN KM  N. LATITUDE OF RIGHT PLOT EDGE, KM

86 200.000000000AN KM  E. LONGITUDE OF RIGHT PLOT EDGE, KM

87 50.0000000000AN KM  DISTANCE BETWEEN RANGE TICKS, KM

88-2.50000000000       HEIGHT OF HORIZONTAL PLOT SECTION ABOVE MSL,KM

90 6.00000000000       TOPOGRAPHY CONTOURS

92 -3.0000000000LN KM  INITIAL CONTOUR VALUE

93 -1.5000000000LN KM  FINAL CONTOUR VALUE

94 0.10000000000LN KM  STEP IN CONTOUR VALUE

96 10.0000000000       DISTANCE BETWEEN CROSS-RANGE TICKS, KM

0                     **********  END OF RUN SET NUMBER 6  *********

N01-7     Sample Case for CONPLT Documentation

29 1.                  1=PROCESS RUNSET, 0=SKIP RUNSET

81 4.00000000000       RAYPLOT PROJECTION PLANE (2 = HORIZONTAL)

82 40.0000000000       PLOT-ORDINATE EXPANSION FACTOR [1.]

85  35.265396   AN KM  N. LATITUDE OF RIGHT PLOT EDGE, KM

86 200.000000000AN KM  E. LONGITUDE OF RIGHT PLOT EDGE, KM

87 50.0000000000AN KM  DISTANCE BETWEEN RANGE TICKS, KM

88-3.00000000000       HEIGHT ABOVE MSL OF BOTTOM OF GRAPH, KM

89 0.0                 HEIGHT ABOVE MSL OF TOP OF GRAPH, KM

90 5.00000000000       CURRENT SPEED CONTOURS

92-1.00000000000LN  M  MINIMUM CONTOUR LEVEL

93 1.00000000000LN  M  MAXIMUM CONTOUR LEVEL

94 0.10000000000LN  M  CONTOUR INTERVAL

96 1.00000000000       DISTANCE BETWEEN DEPTH TICKS, KM

0                     **********  END OF RUN SET NUMBER 7  *********

Appendix C4.  Input Data File Listing for Sample Case 4

GEORGES  RL-3    X6437

EL1              EL NINO MODEL FOR EIGEN PROGRAM - GAP SPECIFIED.

29      0.             DO EIGENRAYS AND BOTH PLOTS

403    -15.0     AN DG  MINIMUM ELEVATION ANGLE

404     15.0     AN DG  MAXIMUM ELEVATION ANGLE

405      2.      AN DG  TICK INTERVAL FOR ELEVATION ANGLE

406     50.             TICK INTERVAL FOR RANGE

407     50.             MAXIMUM HOPS FOR RANGE-ELEV PLOT

408    160.             MAXIMUM NUMBER OF RAYSETS TO PROCESS

409     -0.2     AN DG  BEGIN ELEVATION ANGLE GAP

410      0.2     AN DG  END ELEVATION ANGLE GAP

428     -3.             MINIMUM TRAVEL TIME, SECONDS

429      0.1            MAXIMUM TRAVEL TIME, SECONDS

430      0.5            TICK INTERVAL FOR TRAVEL TIME, SECONDS

432     50.             MAXIMUM HOPS FOR RANGE-TIME PLOT

433    160.             MAXIMUM NUMBER OF RAYSETS TO PROCESS

434      1.485          REFERENCE SOUND SPEED FOR RANGE-TIME PLOT

435      1.             PLOTTING UNITS FOR TIME SCALE (1=SEC)

436    300.             MAXIMUM RANGE FOR BOTH PLOTS

437    100.             MINIMUM RANGE FOR BOTH PLOTS

453    290.             RANGE FOR WHICH EIGENRAYS ARE DESIRED

454      0.001          ELEVATION/RANGE SLOPE CUTOFF

456     50.             MAXIMUM HOPS FOR EIGENRAY INTERPOLATION

457    160.             MAXIMUM NUMBER OF RAYSETS TO PROCESS

458      2.             RUNSET TO USE FROM TAPE1

000

Appendix C5.  Input Data File Listing for Sample Case 5

MIKE=JONES=RL3=X6464

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

3-1.20000000000       TRANSMITTER HEIGHT, KM

4 0.           AN KM  TRANSMITTER POSITION, LATITUDE, KM

5 00.          AN KM  TRANSMITTER LONGITUDE, KM

7 400.000000000FQ HZ  INITIAL FREQUENCY, HZ

11 90.0000000000AN DG  AZIMUTH ANGLE

15-14.0000000000AN DG  INITIAL ELEVATION ANGLE, DEG

16 14.0000000000AN DG  FINAL   ELEVATION ANGLE, DEG

17 .200000000000AN DG  STEP IN ELEVATION ANGLE, DEG

20-1.10000000000       RECEIVER HEIGHT

21 0.                  PENETRATING RAYS NOT WANTED(=0)

22 50.0000000000       NUMBER OF HOPS

26 0.                  MAXIMUM RAY HEIGHT

27-5.00000000000       MINIMUM RAY HEIGHT

28 1000.00000000       MAXIMUM RANGE

28 1100.00000000       MAXIMUM RANGE

29 1111111.00000       ENABLE FUNCTIONS DESIRED

42  .1000000E-07       SINGLE STEP ERROR

44 .100000000000       INITIAL INTEGRATION STEP SIZE

150 5.00000000000       CSMUNK1 SOUND SPEED MODEL

152 2.00000000000       INPUT DATA SET ID NUMBER

153 1.00000000000       REFERENCE SOUND SPEED

154 0.           AN KM  PH1 LONGITUDE 1

155 1.49200000000       CA1 SOUND SPEED ON AXIS

156-1.30000000000       ZA1 DEPTH OF AXIS

157 1.30000000000       H1 SCALE DEPTH

158 .007400000000       EP1 FRACTIONAL INCREASE OF C WITH DEPTH

159 1000.00000000AN KM  PH2 LONGITUDE 2

160 1.49200000000       CA2 SOUND SPEED ON AXIS

161-1.30000000000       ZA2 DEPTH OF AXIS

162 1.30000000000       H2 SCALE DEPTH

163 .007400000000       EP2 FRACTIONAL INCREASEE OF C WITH DEPTH

275 1.00000000000       RHORIZ RECEIVER MODEL CHECK NUMBER

300 1.00000000000       GHORIZ TERRAIN MODEL CHECK NUMBER

302 2.00000000000       INPUT DATA SET ID NUMBER

303-5.00000000000       CONSTANT TERRAIN HEIGHT

350 1.                  SHORIZ OCEAN SURFACE MODEL CHECK NUMBER

375 0.                  NPSURF OCEAN SURFACE PERTURB MODEL CHECK NUM

57 0.                  INTEGRATE AND PRINT PHASE PATH

60 0.                  INTEGRATE AND PRINT GEOMETRICAL PATH LENGTH

71 100.000000000       PERIODIC PRINTING INTERVAL

72 1.00000000000       OUTPUT RAYSETS

73 1.                  TURN ON DIAGNOSTIC PRINTOUT

74 0.00000000000       SUPPRESS ALL PRINTOUT FROM SUBROUTINE PRINTR

75 .100000000000       FULANN HEIGHT OF LETTERING IN INCHES

80 0.0                 NORMAL RAYPATH PLOTS

81 -3.                 PLOT PROJECTION OF RAY (1=VERT.,2=HORIZ) PLANE

82 10.0000000000       PLOT EXPANSION FACTOR

83 0.0          AN KM  LEFT LATITUDE OF PLOT, KM

84 900.         AN KM  LEFT LONGITUDE OF PLOT, KM

85 0.           AN KM  RIGHT LATITUDE OF PLOT, KM

86 1000.00000000AN KM  RIGHT LONGITUDE OF PLOT, KM

87 10.0000000000AN KM  DISTANCE BETWEEN TIC MARKS, KM

88-5.00000000000       HEIGHT ABOVE GROUND OF BOTTOM OF GRAPH, KM

89 0.                  HEIGHT ABOVE SEA LEVEL OF THE TOP OF THE GRAPH

17 0.           AN DG  STEP IN ELEVATION ANGLE

29 0.                  PICK DESIRED FUNCTIONS

15 -13.867      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15 -13.503      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15 -12.167      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15  13.480      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15 -11.770      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15  13.101      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15 -10.455      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15  11.741      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15 -10.005      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15  11.322      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15  -8.650      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15   9.968      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15  -8.110      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15   9.483      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15  -6.600      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15   8.056      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15  -5.885      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15   7.456      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15  -3.802      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15   5.784      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15  -2.558      AN DG  ELEVATION ANGLE

0

0T5    MUNK'S 1974 UNIFORM SOUND CHANNEL

15   4.933      AN DG  ELEVATION ANGLE

0

Appendix D.  FORTRAN Format Specification for Plotting Commands

Part I.  Commands Used by PSGRAPH

Command

0 -  User Identifier Number of Lines:    2

Contents of Line 1: "0"

Contents of Line 2: Text

Format of Line 1:   I2

Format of Line 2:   A

1 -  Moveto with "pen up" to IX, IY

Number of Lines:    Variable (up to 377)

Contents of Line 1:  "1" IX  IY

Contents of Line 2:  Number of pairs of points to follow

Contents of Lines 3 to 377:  8 pairs of X,Y plot coordinates

Format of Line 1:   I2 2I5

Format of Line 2:   I5

Format of Lines 3 to 377:  8(2I5)

NOTE:     If number of points on line 2 is not evenly divisible by 8,

then the remaining pairs of points are each written to a

separate line (1 pair per line).

3 -  Text for Ray Plots or Text for Profile Subtitles

Number of Lines:    3

Contents of Line 1:  "3" IX  IY location for characters

Contents of Line 2:  Orientation, number of characters

Contents of Line 3:  Text

Format of Line 1:   I2 2I4

Format of Line 2:   2I3

Format of Line 3:   A

4 -  Sequence of Moveto and Lineto Commands

Number of Lines:    Variable

Contents of Line 1:  "4"

Contents of Line 2:  Number of Pairs of Points to be Plotted

(there are actually twice this number of pairs of points since

there is one pair for the moveto and one pair for the lineto.)

Contents of Lines 3 to N: 8 pairs of X,Y plot coordinates

Format of Line 1:   I2

Format of Line 2:   I4

Format of Line 3:   8(2I5)

NOTE:  If number of points on line 2 is not evenly divisible by 8,

then the remaining pairs of points are each written to a separate line

(2 pairs per line, one moveto pair and one lineto pair).

-2 - Frame Advance for Ray Plots

Number of Lines:    1

Contents of Line 1:  "-2"

Format of Line 1:  I2

-1 - End of Plotting for Ray Plots

Number of Lines:    1

Contents of Line 1:  "-1"

Format of Line 1:  I2

22 -    Lower Left Origin (in inches from lower left corner)

Number of lines:    1

Contents of Line 1:  "22"  X Y

Format of Line 1:  I2 2(F5.2)

25 -    Text for Title and Axis Labels

Number of Lines:   4

Contents of Line 1:  "25"  X-axis length, Y-axis length (inches)

Contents of Line 2:  Title length, title

Contents of Line 3:  X-axis label length, X-axis label

Contents of Line 4:  Y-axis label length, Y-axis label

Format of Line 1:  I2 2(F6.2)

Format of Line 2:  I5 1X A40

Format of Line 3:  I5 1X A40

Format of Line 4:  I5 1X A40

29 -    Points to be Plotted for a Curve

Number of Lines:   Variable

Contents of Line 1:  "29", Number of points, Symbol Marker

Contents of Line 2 to N:  x(i), y(i), x(i+1), y(i+1) where

i = 1 to Number of points

Format of Line 1:  I2, 2(I5)

Format of Line 2 to N:  4(E14.6)

NOTE:  If the number of points is odd, then the last line has only one

x,y pair.  Thus, the Nth line would have format 2(E14.6).

30 - Frame Advance for Profiles

Number of Lines:  1

Contents of Line 1:   "30  1"

Format of Line 1:  I2, I3

31 -    End of Plotting

Number of Lines:  1

Contents of Line 1:   "31"

Format of Line 1:  I2

39 - Maximum, minimum and interval values for both axes in real units

Number of Lines:  1

Contents of Line 1:  "39", X-axis minimum, X-axis interval,

X-axis maximum, Y-axis minimum, Y-axis interval, Y-axis maximum

Format of Line 1:  I2, 6(E13.7)

40 - Indicates Eigen plot is being produced

Number of Lines:  1

Contents of Line 1:  "40"

Format of Line 1:  I2

Appendix D.  FORTRAN Format Specification for Plotting Commands

PART II.  Commands Used by DISSPLA (Ignored by PSGRAPH)

10 - Font indicator

Number of Lines:  1

Contents of Line 1:  "10"

Format of Line 1:  I2

11 - Case indicator

Number of Lines:  1

Contents of Line 1:  "11 STAND      !"

Format of Line 1:  I2,trl,a,1x,al

12 - Case indicator

Number of Lines;  1

Contents of Line 1:  "12 L/CSTD     #"

Format of Line 1:  I2,trl,a,lx,al

13 - Text Height

Number of Lines:  1

Contents of Line 1:  "13" number

Format of Line 1:  I2,F4.2

20 - Initializes DISSPLA to create an intermediate compressed output

file

Number of Lines:  1

Contents of Line 1:  "20"

Format of Line 1:  I2

21 -    Clipping Indicator

Number of Lines:  1

Contents of Line 1:  "21" number

Format of Line 1:  I2,F4.2

23 -    Page Limits

Number of Lines:  1

Contents of Line 1:  "23" number, number

Format of Line 1:  I2, 2F6.2

24 -    Scale Factor for plot symbols

Number of Lines:  1

Contents of Line 1:  "24" number

Format of Line 1:  I2,F5.2

32 -    Number of X axis ticks

Number of Lines:  1

Contents of Line 1:  "32" number

Format of Line 1:  I2,I3

33 -    Number of Y axis ticks

Number of Lines:  1

Contents of Line 1:  "33" number

Format of Line 1:  I2,I3

35 -    For converting negatively valued labels to positive numbers.

Begin converting at the "mth" number and continuing for "n"

numbers.

Number of Lines:  1

Contents of Line 1:  "35" m,n

Format of Line 1:  I2,2I3

36 -    Suppress border

Number of Lines:  1

Contents of Line 1:  "36"

Format of Line 1:  I2

TABLE 1.  -  Plotting Commands in TXTOUT Files Used by PSGRAPH

RAYPLOTS

0 Frame start

1    Moveto with linetos

3    Text location + text

-2    Frame advance

-1    End of plotting

CONTOURS

0    Frame start

1    Moveto with linetos

3    Text location + text

4    Alternating moveto,linetos

-2    Frame advance

-1    End of plotting

PROFILES

22    Lower left origin (in inches from corner)

25    Title Text and Axis Labels

29    Points to be plotted

30    Frame advance

31    End of plotting

39    Coordinates for plot (in plotting units)

EIGEN PLOTS

3   Text location + text

22    Lower left origin (in inches from corner)

25    Title Text and Axis Labels

29    Points to be plotted

30    Frame advance

31    End of plotting

39    Coordinates for plot (in plotting units)

40   Indicates Eigen plot

TABLE 2. Plotting Commands in TXTOUT Used by DISSPLA Software

(Ignored by PSGRAPH)

10    Font indicator

11    Case indicator

12    Case indicator

13    Text Height

20 Initializes DISSPLA to create an intermediate

compressed output file

21    Clipping Indicator

23    Page Limits

24    Scale Factor for plot symbols

32    Number of X axis ticks

33    Number of Y axis ticks

35    For Converting Negative Labels to Positive

Numbers

36    Suppress border